To:	Mark V Walker/EWC/Enron@ENRON

cc:

bcc:

Subject:	Conditons that increment "Turbine OK" timer

Hi Mark,

I check on the issues that you raised the other day. The "manual mode" conditions (manual yaw or pitch command, manual generator speed or torque settings, etc.) increment the "maintenance timer" not the "turbine OK" timer.

There is one function that will cause the "OK" timer to increment when the turbine is capable of producing but is not producing. If the "load shut down" command is given to the turbine, it will display "147 Load shutdown" status message. If the turbine is not faulted (due to some other condition) the "turbine OK" timer will increment. It is possible that some people in the field use this command to take the turbine "off-line" without effecting the availability.

Hope this helps,

Garth