ARIZONA DEPARTMENT OF ENVIRONMENTAL QUALITY

AIR QUALITY CLASS I PERMIT

SIGNIFICANT REVISION NO. 1001602

TO AIR QUALITY CONTROL PERMIT NO. 1000155

For

TRANSWESTERN PIPELINE COMPANY

FLAGSTAFF COMPRESSOR STATION

SIGNIFICANT REVISION PERMIT DESCRIPTION

This Significant Revision  is issued to the Transwestern Pipeline Company, the Permittee, for the replacement of three reciprocating 4000 hp Ingersoll-Rand 616KVT engines with a single General Electric (GE) LM2500 gas turbine and the replacement of two existing 451 horsepower (hp) Waukesha LRZ3520GU natural gas generator engines with two 526 hp Caterpillar G3508 natural gas generator engines. However, only one of two Caterpillar generators can be operated at a time. The GE gas turbine is site rated at 30,565-shaft horse power (shp) at 600F under base load conditions. The Flagstaff Compressor Station is located 3 miles west of Flagstaff in Coconino County, Arizona. The existing facility is permitted by Title V Permit, 1000155, which was issued on April 16, 1998 and valid until April 16, 2003.

This facility is an existing major source as defined by the Prevention of Significant Deterioration (PSD) program, Arizona Administrative Code (A.A.C) R18-2-401 et. seq. However, it is a Agrandfathered source@ because the compressor engines were installed in 1966, before the PSD program was established. No changes have been made at the facility since installation. After this revision, PSD will still not be applicable because the emissions from the turbine and generator will be below PSD major source levels.

The PSD regulations are not applicable to the proposed modification itself because the Permittee has shown that the difference between the emissions from the new equipment and the decrease in emissions from removal of the existing equipment is below the significance levels defined in the Arizona Administrative Code (A.A.C). R18-2-104.a. The project, therefore, qualifies as a Significant Revision according to the provisions of A.A.C.R18-2-320.

Attachment AA@ below shall replace Attachment AA@ of Permit No. 1000155 because ADEQ has updated the general provisions since the issuance of the Title V permit.

ATTACHMENT "A": GENERAL PROVISIONS

ATTACHMENT "B": SPECIFIC CONDITIONS

Condition V below shall be added to Attachment AB@ of Permit No. 1000155

V. 	Transition Period 		[A.A.C. R18-2-306.01]

The Permittee shall comply with the following operating scenario after the startup of the General Electric turbine, Unit 204:

A. 	Operating Schedule

1. 	The transition period for this project shall be six calendar months after the startup of new Unit 204. Within 15-calendar days, the Permittee shall provide written notification to the Director upon startup of Unit 204.

[A.A.C. R18-2-306.A.11]

2. 	During the transition period, new Unit 204 shall not be operated concurrently with existing Units P1, P2 or P3.

3. 	During the transition period, only one of the two existing generator engines (P4 or P5) can be operated concurrently with one of the new generators (223 or 224).

4. 	After the transition period, existing Units P1, P2, P3, P4 and P5 shall be permanently shutdown. The Permittee shall provide a written notification to the Director of the permanent shutdown of the Units.

B. 	Operational Limit		       [A.A.C. R18-2-306.A.2, -306.01, and -331]

During the transition period, each of the existing equipment, P1, P2, P3 and P4/P5 shall not be operated for more than 3100 hours.

[THIS IS A MATERIAL PERMIT CONDITION]

C. 	Recordkeeping 	[A.A.C.R18-2-306.A.3.b]

During the transition period, the Permittee shall maintain logs of daily operating hours for Units P1, P2, P3 and P4/P5.

Condition VI below shall be added to Attachment AB@ of Permit No. 1000155

VI. 	GE LM2500 Gas Turbine	, Unit 204

A. 	Nitrogen Oxide

1. 	Emission Limitations/Standards	[40 CFR '60.332]

a. 	The Permittee shall not cause to be discharged into the atmosphere from the stack of any turbine, any gases which contain NOx in excess of that calculated from the following equation:

STD =  0.0075    (14.4)    +  F

Y

Where:

STD =	allowable NOx emissions (percent by volume at 15 % O2 and on a dry basis).

Y  =	manufacturer's rated heat rate at manufacturer's rated peak load (kilojoules per watt hour), or actual measured heat rate based on lower heating value of fuel as measured at actual peak load for the facility.  The value of Y shall not exceed 14.4 kilojoules per watt hour.

F  =	NOX emission allowance for fuel-bound nitrogen as defined in 40 CFR '60.332(a)(3).

2. 	Testing Requirements 	[40 CFR '60.335.(c).(2), A.A.C. R18-2-312]

a. 	Within 180 days of startup, the Permittee shall conduct or cause to be conducted, an initial performance test at peak load conditions and furnish the Director a written report of the results of the test.

b. 	After the initial performance test, subsequent performance tests shall be conducted on an annual basis under peak load conditions. The Permittee furnish the Director a written report of the results of the test.

B. 	Carbon Monoxide	[A.A.C R18-2-306.01, 331.A.3.a]

1. 	Emission Limitations/Standards

The emissions of CO shall not exceed 40 ppmvd at 15% O2 based on a 3-hour average.

[A.A.C R18-2-306.01, -331]

2. 	Testing Requirements 	[40 CFR '60.335.(c).(2), A.A.C. R18-2-312]

a. 	Within 180 days of startup, the Permittee shall conduct or cause to be conducted, an initial performance under peak load conditions and furnish the Director a written report of the results of the test.

b. 	After the initial performance test, subsequent performance tests shall be conducted on an annual basis under peak load conditions. The Permittee furnish the Director a written report of the results of the test.

C. 	Fuel 	[A.A.C. R18-2-306.A.2]

1. 	The Permittee shall only fire pipeline quality natural gas in the turbine.

2. 	The Permittee shall keep records of the natural gas usage and maintain a vendor-provided copy of that part of the Federal Energy Regulatory Commission approved tariff agreement that contains the sulfur content and lower heating value of the pipeline quality natural gas.

Condition VII below shall be added to Attachment AB@ of Permit No. 1000155

VII.	Caterpillar G3508 Natural Gas Generator Engine, Unit 223 and Unit 224

A.	Emission Standards	             [A.A.C R18-2-719.B, -719.C]

1.	Particulate Matter

a. 	The Permittee shall not cause, allow or permit the emission of particulate matter, caused by combustion of fuel, from any internal combustion engine stack having a heat input rate of 4200 million BTU per hour or less, in excess of the amount calculated by the following equation:

E = 1.02Q 0.769

where:

E = the maximum allowable particulate emissions rate in pounds-mass per hour

Q = the heat input in million BTU per hour

For the purposes of this condition, the heat input shall be the aggregate heat content of all fuels whose products of combustion pass through the engine stack or other outlet.  The total heat input of all operating fuel burning units on a plant or premises shall be used for determining the maximum allowable amount of particulate matter which may be emitted.

b.	The Permittee shall not cause, allow or permit to be emitted into the atmosphere from the generator, smoke which exceeds 40 percent opacity, for any period greater than ten consecutive seconds.  Visible emissions when starting cold equipment are exempt from this requirement for the first ten minutes.

[A.A.C. R18-2-719.E]

c.	The Permittee shall not burn in the engines any fuel which contains sulfur in excess of 0.8 percent by weight. 	[A.A.C. R18-2-719.J]

B.	Fuel Limitations							        [A.A.C. R18-2-306.A.2]

The Permittee shall only fire pipeline quality natural gas in the generator.

C.	Operational Limitations					[A.A.C. R18-2-306.A.2, -306.01, and -331]

1.	The Permittee shall only operate one of the two generators at any time.

[THIS IS A MATERIAL PERMIT CONDITION]

2.	The Permittee shall limit the hours of operation of the generator to 7800 hours for any consecutive 12-month period.

[THIS IS A MATERIAL PERMIT CONDITION]

D.	Testing Requirements 	[A.A.C.R18-2-311, -312]

    - Within 180 days after initial start-up, the Permittee shall conduct initial performance tests for CO and NOx on the generator. The Permittee shall furnish a written report of the results of the tests to the Director.

2.	The Department may deem it necessary to conduct a performance test based on site/operation inspection and excess emissions.  If testing is required, Permittee shall meet the requirements described in Section XVIII of Attachment "A". The performance test shall be conducted and data reduced (as required by A.A.C. R18-2-312.B) in accordance with the following test method and procedures contained in the Arizona Testing Manual or 40 CFR Part 60, Appendix A:

a.	EPA Reference Method 7E for NOx.

b.	EPA Reference Method 25 for CO.

c. 	EPA Reference Method 9 for the opacity of visible emissions.

d. 	EPA Reference Method 5 for particulate matter.

E.	Periodic Monitoring 	[A.A.C.R18-2-306.A.3]

After the initial performance test, the Permittee shall conduct subsequent EPA Method 9 readings to determine compliance with opacity standards on a quarterly basis.

F.	Recordkeeping and Reporting Requirements 	[A.A.C.R18-2-306.A.3.b, -719.I and -719.J]

1.	Fuel

The Permittee shall keep records of the natural gas usage and maintain a vendor-provided copy of that part of the Federal Energy Regulatory Commission approved tariff agreement that contains the sulfur content and lower heating value of the pipeline quality natural gas.

2.	Hours

a. 	The Permittee shall maintain monthly records of the hours of operations of the generator.

b. 	The Permittee shall maintain 12-month rolling totals (i.e., sum of the current month and the previous 11 months) of the operating times of the generator.

3.	Opacity

The Permittee shall maintain records of all quarterly opacity readings in a log.  The record shall have at a minimum the name of the observer, the date and time of opacity test, the source on which the test is being performed and the results of the test.

4. 	The Permittee shall notify the Director in writing within 30 days of any changes to the FERC-approved Tariff agreement relating to the fuel sulfur content and lower heating value limits that occur during the term of this permit.	[A.A.C. R18-2-306.A.5.a]

ATTACHMENT AC@

The following shall be added to the list of applicable requirements in Attachment AC@ of Permit No. 1000155.

ARTICLE 7		EXISTING STATIONARY SOURCE PERFORMANCE STANDARDS

R18-2-719.A, B, C	Standards of Performance for Existing Stationary Rotating Machinery

E, I, J

ARTICLE 9		NEW SOURCE PERFORMANCE STANDARDS

R18-2-901.1		40 CFR 60, Subpart A, General Provisions

R18-2-901.40	40 CFR 60, Subpart GG

ATTACHMENT AD@

The following equipment shall be added to the list of equipment in Attachment AD@ of Permit No. 1000155. Six months after startup of Unit 204, existing equipment identified as P1, P2 and P3 in Permit 1000155 shall be permanently retired.

| Emission Unit ID | Description | Manufacturer | Fuel Type | Size | Serial Number | Model | Date of Manufacture |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 204 | Compressor turbine | General Electric | Natural gas | 30565 hp @ 60 F (Base load) |  | LM2500 | 2001 |

Only one of the following equipment can be operated at a time. The following equipment shall replace the equipment identified as P4 and P5 in Attachment AD@ of Permit No. 1000155. Equipment P4 and P5 shall be permanently retired after issuance of this Significant Revision.

| Emission Unit ID | Description | Manufacturer | Fuel Type | Size | Serial Number | Model | Date of Manufacture |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 223 and 224 | Generator engines | Caterpillar | Natural Gas | 526 hp |  | G3508 | 2001 |