PLUG & ABANDONMENT PROCEDURE

FOR

NAPOLEONVILLE STORAGE #1

WELL DATA:

# TUBLARS		WEIGHT		DEPTH		DESCRIPTION

16”			122.15 #/FT.		2,009 FT.		LAST CEMENTED

CASING

8 5/8”		  36 #/FT.		3,211 FT.		HANGING STRING

PROCEDURE:

- Submit Work Permit to Department of Natural Resources (DNR).  Contact DNR at least 24 hours prior to plugging operations.

- Displace natural gas by filling cavern with brine or freshwater.

- Circulate until gas is cleared from wellbore.

- Rig up workover rig and remove top section of the wellhead components.

- Rig up preventers and pull hanging string.

- Pick up bridge plug and set at 2,000 feet.

- Fill up 16” casing from the bridge plug to the surface with cement, using the circulation method.

- Cut the 16” off approximately 3 feet below ground level and plate over.  Weld into the plate the Serial Number of the well and the date.

- Prepare and submit appropriate government reports, Form P & A.