Assume a unit has submitted the following information for 3 points, their Pmin is 75 and Pmax is 300:

MW		Heat Rate		ER

100		13000			0.8

200		11000			0.7

300		10500			0.6

The price of gas is $9, the emission price is $0.

The formula we will use is:

Ci'=maxCi−1', 0.001 HRi+1 Qi+1−HRi QiQi+1−Qi PGas+ERi+1 Qi+1−ERi QiQi+1−Qi PNOx+α∴ i=1,2,…,n−1Qi≤Q≤Qi+1C0'=0

αCi is the cost at point i, Qi is the output at point i, for n number of points. is the fixed cost adder, equal to $2.

Note: If the cost resulting from the heat rate portion or the emissions portion of the equation is negative, the portion will be adjusted to zero.

We first calculate the cost at the first point:

C1=max0, 0.001 11000x200−13000x100200−100 9+0.7x200−0.8x100200−100 0+2=$83

Next, we calculate the cost at 2nd point:

C2=max83, 0.001 10500x300−11000x200300−200 9+0.6x300−0.7x200300−200 0+2=$87.50

Next, to cover the unit’s full range of operation, we would extend the curve down to Pmin by setting Q1= 75.

Finally, to convert the results into a bid, we would set C3 = C2.  The resulting MW-price pairs are as follows:

( 75, $83.00 )

( 200, $87.50 )

( 300, $87.50 )