ABNORMAL OPERATION REPORT

| Date of Abnormal Operation: | 1/5/02 |
| --- | --- |
| Date of Report: | 1/9/02 |

PERSON SUBMITTING REPORT TO COMPANY REPORTING UNIT

| Name: | Rick Watts |
| --- | --- |
| Job Title: | SR. O&M TECH |
| Business Telephone: | 651-982-2162 |

LOCATION OF CONDITION

| Region: | MPLS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location: | North Branch |  |  |  |  |  |
| Operating Company: | Northern Natural Gas Co., |  |  |  |  |  |
| Pipeline Name: |  |  |  | Number: |  |  |
| Station or Mile Post: |  |  |  |  |  |  |
| Compressor/Plant: | North Branch Compressor Sta. |  |  |  |  |  |
| State: | Mn. |  |  |  |  |  |
| County/Parish: | Chisago |  |  |  |  |  |
| Section:4 | 35n |  | 21w |  |  | Survey: |
| Nearest Road or Street Address (where applicable): |  | Forest Blvd. 420th St. |  |  |  |  |

SIGNIFICANT DETAILS OF CONDITION

10:22 unit #2 on line, units 2&3 relief valves blowing

Sta. code 4 chi. Code 4

Pressures were 1022 north set point 1040

763 south 750

908 west 910

Step 13 rpm 250

PERSONNEL NOTIFIED

| Control Center: | North Branch Police responded to homeowner call of noise at the plant. They called Occ and they called |
| --- | --- |
| Supervision: | the duty pager. |
| Other Location(s): |  |

PERSONNEL AND EQUIPMENT DISPATCHED

Rick Watts responded to the call by OCC.

I notified Gas Control that we needed to shut down Eng. #2 as the reliefs were blowing.

We were going to by-pass the sta. and as we were waiting for the pressures to equalize the pressure started rolling back into Farmington. Set points were set to the maximum we could go except for the north, which was set at 1015 and #3 Eng. was put on. It was later shut down at 03:30 for no demand.

ACTIONS TAKEN

Relief pilots on units 2&3 were taken apart inspected, cleaned, one new “o” ring was replaced in #2 pilot reassembled and installed. Both reliefs’s were tested with nitrogen and passed. Unit #3 pilot was dirty with rust and scale.unit #2 had a small amount of oil in it.

We are looking into prices to replace pilots on units 1,3,4, as these are appx.30 yr. old. Appx. $3300.00

ANALYSIS AND POSSIBLE ACTIONS TO PREVENT RECURRENCE

Install inline filters in pilot relief sensing line.

REVIEW PER OP 10.108

| Date: | 1/9/02 |
| --- | --- |
| Participants: | North Branch Team |
| Comment(s): |  |