Status of Electric Utility Industry Deregulation

In Japan

Prepared for Korea Energy Economics Institute

August 16th, 1999

Mitsubishi Research Institute, Inc.

INDEX

1. Concept of Deregulating Retail Market	2

2. Status of Partial Retail Deregulation	3

2.1 Agenda at Electric Utility Council	3

2.2 Scope of Retail Deregulation	4

2.3 Providing Last Resort	6

2.4 Protection from Cross subsidy	7

2.5 Rule of Retail Wheeling	8

2.6 Detailed Rule of Retail Wheeling	10

2.7 Eliminating Monopoly Power	15

2.8 Aggregation	17

2.9 Wheeling-out, Self-wheeling	18

3. Prospects for Further Deregulation	19

# Concept of Deregulating Retail Market

1) Customers connected to over 20,000V (HHV Class) can purchase electricity from Private Power Suppliers (PPSs) other than conventional electric utilities.  - "Partial Deregulation"

2) PPSs can use utilities' transmission grid to wheel their electricity directly to the customers.

3) Definition of "one customer" was expanded.

G

G

IPP

IPP

Consumption

PPS

Nuclear

Large scale thermal Plant

500kV Transmission line

Thermal Plant at Suburban area

HHV Transformer Station

250-100kV Transmission line

HV Transformer Station

70-20kV Transmission line

Private Line

Distribution Transformer Station

Retail Wheeling

Private Generator

Self

Consumption

6KV Distribution line

HHV Customers

# Status of Partial Retail Deregulation

## Agenda at Electric Utility Council

- Scope of retail deregulation		What should be the definition?

- Providing last resort			How to secure electricity supply to such customers that could

not reach agreement with any PPSs, or the electric utilities.

Such customers should be provided certain protection by the regulation.

- Protection from cross-subsidy 		Certain protection has to be facilitated in utility regulating

system to eliminate the cross-subsidy, that may occur between unregulated section and regulated section in the utility company that needs to gain its competitive position in the unregulated market.

- Rule of retail wheeling			What would be the fair transmission access rule and wheeling rate?

- Eliminating monopoly power		How to secure a fair competitive market from a view point of

relationship among PPSs and relationship between suppliers and large customers.

- Improve rate-making rule			How to propagate benefit obtained from partial deregulation to all

the ratepayers.

- Enhance wholesale competition		How to harmonize new retail competition and previously-

introduced wholesale bidding system.

## Scope of Retail Deregulation

Interim report of Electric Utility Council （1999.1.12）

Recent discussion at the Council and forecast

1) HHV class customers connecting to 20kV line that has over 2000kW demand.

2) Definition of "one customer"

・"One premise" definition.

・Collaboration of demands by private lines

- Open to HHV customers until the review after 3 years.

- Whether deregulation expands to HV customer will be an issue at the review.

- "One premise" requires no accounting separation but obvious physical separation such as hedge or wall.

- "Collaboration" requires self-governing, community-like characteristics (e.g. industrial complex, small and medium-sized industrial park, regional development, etc.)

Private line

HHV line

HV line

HHV line

obvious physical separation

Company A

Factory

Company B

Factory

Two customer

Company A

Factory

Company B

Factory

"One customer"

Expansion of "One premise" definition

Company A

Factory

Company B

Factory

Two customer

"One customer"

Company A

Factory

Company B

Factory

Expansion of " Collaboration " definition

Table 2-1　Definition of "one customer"

|  |  |  | Correspondence to the definition of "one customer" | Regulation to construct private power line |
| --- | --- | --- | --- | --- |
| One premise |  |  | Yes | Unregulated |
| Two or more | Collaboration |  | Yes | Unregulated |
|  | Non-collaborated customers | Special relationship among customers | No | Regulated - Special supply Permission required |
|  |  | No special relationship among customers | No | Regulated - Special area utility Permission and rate regulation |

## Providing Last Resort

Interim report of Electric Utility Council （1999.1.12）

Recent discussion at the Council and forecast

1) Utilities should provide last resort service tariff approved by MITI

2) The tariff can include a provision securing that utility do not obligate to supply electricity when the utility do not have enough reserve margin.

- MITI can order utility to change the provision of the last resort tariff, when MITI found the necessity to keep fair competitive market, protection of the customer from monopoly.

- Rate level of the last resort supply is not clear.

- The tariff will be applicable to PPS's self-consumption.

Last resort supply

Consumption

Generator

Consumption

Consumption

Consumption

Generator

A　PPS

B Customer

PPS business terminate.

B Customer

## Protection from Cross Subsidy

Interim report of Electric Utility Council （1999.1.12）

Recent discussion at the Council and forecast

1) A reasonable framework of accounting separation should be developed and legislated.

2) Utility should notice the method of cost allocation to MITI. The method should be publicly announced.

- Transmission cost should be separated between regulated and unregulated sector using the existing cost allocation rule.

- C.P.A and MITI examine the cost allocation.

Existing allocation rule

ABC* accounting

|  |  | Regulated Sector |  | Unregulated Sector |  |  | Wheeling Sector （regulated） |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Generation |  |  |  |  |  |  |  |  |  |
| Transmission |  |  |  |  |  |  |  |  |  |
| Transformation（HHV） |  |  |  |  |  |  |  |  |  |
| Transformation（< HHV） |  |  |  |  |  |  |  |  |  |
| Distribution |  |  |  |  |  |  |  |  |  |
| Marketing |  |  |  |  |  |  |  |  |  |
| Management |  |  |  |  |  |  |  |  |  |

↓		　　　　　　　　↓	　　　　↓

*ABC: Activity Based Cost

Regulated Rate		Rate unregulated　　Wheeling Rate

## Rule of Retail Wheeling

Interim report of Electric Utility Council （1999.1.12）

Recent discussion at the Council and forecast

1) Notice of pro-forma wheeling tariff to MITI

Based on the pro-forma tariff, PPSs can negotiate contractual condition of the wheeling with utility company. Arbitration will be made by MITI.

2) Basic rule of the retail-wheeling rate

- "Full recovery of the cost"

But the cost should be just and reasonable.

C.P.A will check the cost calculation.

- "Equitability" between PPSs and utilities

- Forward-looking cost (existing rule)

- Secure recovery of fair return

- Appropriation of the profit from wheeling sector by the utility company.

- No accounting separation in the transmission sector.

- Postage stamp rate in zone

- No discrimination by connecting voltage

- Demand and Energy rate

- Utility can provide special rate tariff. (e.g. TOU, IR, etc. to improve load factor)

- Congestion charge, premium to generator near urban area.

- The burden of the cost of the power supply line and the load line as in the situation to date

Interim report of Electric Utility Council （1999.1.12）

Recent discussion at the Council and forecast

- Consideration for the system stability

PPSs should follow the load-dispatch instruction of the electric power company to maintain the grid stability. For instance, it is suitable that the PPS observes the following matter:

- Submitting feeding power plan which is requested from a view point of system stability

- Balancing the supply to the load ("Load Balancing")

- Following priority load-dispatch instruction in accident, emergency, and securing of water power and nuclear power operation

- Basic rule for Load Balancing

30 minutes accumulated supply should be balanced 30 minutes accumulated demand within 3% error.

Utility have to supply ancillary service to balance the load with compensation by regulated rate for less then 3% error, by unregulated rate over the 3% error. (See below for further details.)

- Condition for priority load-dispatch

(See below for further details.)

## Detailed Rule of Retail Wheeling

#### (1) Rule for Load Balancing

A PPS should supply electricity that balance to the demand of

the PPS's customer at any instance.

In realty, it is impossible to confirm such load balancing.

30 minutes accumulated load and supply should be balanced.

30min

30min

Demand

Supply

Load balancing

:　Ancillary service

#### (2) Retail Wheeling Rate

Level of rate

Retail Wheeling Rate = Transmission Usage Charge + Ancillary Service Charge + Tax for Electricity Development (existing tax ; \ 0.445/kWh)　=  about \3.0 / kWh expected

Rate structure

Standard Rate Tariff				Special Tariff (Incentive for improving L/F)

Capacity charge & Energy charge	Time of Use Rate, Interruptible Rate

Penalty for wheeling capacity excess		:　1.5 times of capacity charge

Premium for closeness to urban area　　　	:　will follow the wholesale bidding system.

Connection to reserve line or spot network　　:　additional charge

#### (3) Load Balancing Charge : LBC

Shortage below 3% :	Energy charge which is noticed to MITI

Shortage over 3%　:	Penalty charge which is no longer regulated

LBC

Excess below 3%	:	Utility may purchase excess electricity by

existing "Excess Electricity Purchase Menu" .

Excess over 3%　	:　　	Utility may pay no charge for excess electricity.

Definition of "3%"	:　3% of the contract wheeling capacity or 3% of the every 30 minutes

load

Exemption　　:	In the case that the load balancing is technically difficult due to rapid growth

or decrease in the demand in such morning hours, PPS could be exempt from

the load balancing charge.

#### (4) Backup Service

Accident in supply or transmission　:	similar to existing backup service for self-generator served by utilities with regulated rate

Periodical inspection of generator　 : 		same as above, but service supplied with

unregulated rate

Backup service menu in the case of load-dispatching by reason mentioned below is under consideration.

Congestion of transmission line		:	No provision expected

Special bottom load time			:	Unknown (TEPCO stated at the council that

there could be very few in a year, so that the dispatch in the case of such bottom load time will not affect the PPSs' business.)

#### (5) Cost Burden of Access Line

- Cost of access line from PPS generator to transmission line, as well as from PPS's customer to transmission line would be basically recovered by wheeling rate, but excess cost over certain level have to be the burden of PPS.

#### (6) Metering, Information Line

- Utility company will set up the 30 minutes meter, monitor the meter, and calculate account billing.

- Information line for the load balancing should be burden of PPS. Utility will prepare an Information line for load-dispatch aside from the load balancing information line, whose cost will be recovered from PPS. So, utility will not provide information on generation power nor PPS's customer demand.

#### (8) Application of Wheeling

- Information to be opened to public will be limited to the power flow data of grid system at the maximum load. Utilities stated, however, on request by PPSs utility will provide detailed grid information to the PPSs, notwithstanding limited information opened to public.

- Utility companies have to establish information security code to assure that information on PPS's wheeling application does not leak from the wheeling section to other section.

- Feasibility for connecting to utility grid will be analyzed by the utility company. When utility find the applied connection to be unfeasible, utility have to show the reason to applicant. If the applicant won't consent, MITI will make arbitration.

## Eliminating Monopoly Power

Complementary supply by utility

PPS supplies electricity for the customer's partial demand, and utility supply for the rest.

Wholesale supply from utility

PPS procures wholesale electricity from utility, mixing the electricity of own generated, the PPS supplies for the total demand of the PPS's customer.

Utility

Customer

Complementary supply

Utility

Wholesale supply

Wholesale transaction

PPS

PPS

Customer

Only one contract for customer

Two contract for customer

No physical difference

Interim report of Electric Utility Council （1999.1.12）

Recent discussion at the Council and forecast

- It is necessary for establishing a rule to eliminate utilities' monopoly power.

- PPSs need utilities' supplementary supply. "As for the refusal of the electric power company of the resale or supplementary supply, there is a doubt which collides with Antimonopoly Law.

- If utility make discount for retaining customer, there is suspect that is against Antimonopoly Law. Utility have to prepare rate menu even for unregulated customers. But menu will be closed.

Utilities' position　　:　　Complementary supply: NO	　Wholesale supply:　OK

Utility grid

PPS supply

Complementary supply

Wholesale supply from utility

wheeling

PPS supply

wheeling

PPS generation

PPS generation

## Aggregation

PPS can aggregate customers and suppliers

Supplier A								Customer A

Supplier B					Wheeling		Customer B

Supplier C		PPS					Customer C

Supplier D								Customer D

Supplier E

- PPS as an aggregator have to make wheeling contract with utility company. In the contract words, PPS have to state all the supplier and the customers.

Wheeling capacity	:	Simple sum of customers demand

Supplemental supply	:	Negotiation with utility company

## Wheeling-out, Self-wheeling

Wheeling-OUT

Utility B

Utility C

Utility A

PPS 							Customer

Wholesale wheeling         Wholesale（Inter-utility）	　　　Retail Wheeling

Reference rate：　　 \0.3		＋　     \0.2		＋　　      \3.0　　　＝	\3.5 / kWh

（including tax）

Self-wheeling	Existing self-wheeling will be continue.

A company

self-generator

PPS B company

A company

Factory

Self-wheeling

Retail wheeling

# Prospects for Further Deregulation

#### (1)　Complete Deregulation of Retail Market and Metering Market

- Complete deregulation of retail market is regarded as a worldwide trend.

- End-users choosing PPS would need to purchase 30-minute metering devices.

- Discussion would involve liberalization of metering and billing. (Metering market is liberalized in the UK, and discussed in California)

- Metering cost for small-scale customers has decreased (to approximately 80 dollars in the US.  Profit will arise from customers of 50kW and above).

- Technical development of electric metering devices (e.g. "Open Planet" in Shikoku Electric)

#### (2)　Establishment of the pool market

- The need for the power pool market was briefly suggested in the Electric Utility Council last year.  However, American and European experiences show that the power pool market may not be suitable for bulk power exchange given the high risk of price fluctuation.  The power pool market is instead handling excess electricity and meeting peak demand in the US and Europe.

Concept of British Power Pool

Peak-load IPP

Bid

National Grid

Sell at contracted price

Mid-load IPP

Purchase at pool price

Pool Price

Providers

Users

Mid-load IPP

Base-load IPP

KW

Base-load IPP

Demand

Contract for Difference (CFD) with the power pool price

Prospect for Electricity Broking

As deregulation allows aggregators to buy and sell excess power, private pool market -- electricity broking within member companies -- may develop, which would be effective in handling excess power and meeting peak demand.  (e.g. US Automated Power Exchange: coordinates electricity futures market in California)

Users of PPS			PPS						PPS		Users of PPS

Users of PPS		      PPS						PPS		Users of PPS

Electricity broker

Users of PPS	       	PPS						PPS		Users of PPS

Self-generator					PPS		Users of PPS

Demand-supply control by price

Purchasing of excess power

Selling of excess power

#### (3) Establishment of the Independent System Operator (ISO)

ISOs are increasing in many countries.

The Electric Utility Council has expressed recognition of a need to establish a fair and neutral system operator.

- However, it is doubtful whether the independence of utilities' dispatch centers is absolutely prerequisite in the context of retail market partial deregulation.

- ISO would be necessary if the power pool market would be established (for the sake of fair management of wholesale electricity), but the pool market is less likely to be realized.

- Moreover, dividing the construction and the management of the transmission network might diminish the incentives for construction, which might be disadvantageous for the healthy development of transmission network.

The Necessity of ISO is marginal

An independent entity assessing grid network is necessary.

Under the status quo, newcomers have no choice but to rely on utilities in deciding optimal siting.

It is necessary also in a sense to guarantee information on newcomers' network request to be contained.

#### (4) Open access of Distribution Network

As distributed generation technology progresses, the point at issue would be the rule for such distributed generators like co-generation, PV, wind mills, waste power, fuel-cell, micro-gas-turbine, connecting to distribution network. Problem of short circuit capacity and islanding operation would arise the limitation of connecting capacity of distributed generators.

Short circuit capacity problem

Short circuit current from utility grid　　　　　　　　 Short circuit current from distributed generators

Disconnecting switch

Co-generation

PV cells

Wind mills

Micro

GT

Short Circuit

Fault

Current overload the disconnecting switch causing facilities of end user break.

End User

#### (5) Future Deregulation Scenario

- In the future, international trend of the electricity market system, entry of foreign capital, and foreign countries' pressure may move the Japanese market to establish a wholesale pool and an independent system operator.

- In that case, privatization of Electric Power Development Company could give an opportunity for further deregulation.

Year				Prospects

2000	Partial liberalization of the retail market starts

:　Emergence of PPS newcomers using retail wheeling.  Number of newcomers limited.

2002	Public sector will establish purchasing procedure for electricity from PPS

Several industrial complexes will start jointly purchasing of power.

Excess power broking will spread among industrial users.

2003	Electric Power Development Company will be completely privatized

EPDC sells coal-fired and hydro power plant?

Foreign energy companies will purchase EPDC's power plants?

2003  	The Electric Utility Council will resume to revise impartial deregulation

Agenda:  Establishment of ISO and power pool market by IPPs

2005	Expansion of liberalization of retail market

Abolishment of the specific supply system

Liberalization of metering and billing

Rise of retail supply

2007	Complete liberalization of retail market