ENERGY EQUIPMENT DATA FOR PULP MILL

Steam Turbine / Generation

# Company: Votorantim Celulose e Papel   Date: 27/08/99   Responsible: Marcelo Castelli

#### Turbogenerator number  1  - ( TG – 1 )

### Steam Turbine

Manufacturer:	Siemens – Dedini

## Serial number:	Impulse & Reaction Type Model 50/40

Year of Commercial Operation: 1994

## Capacity: 			25,340 kW

Number of operations hours: ~ 52.000 equivalent hours

Counterpressure

## Pressure:			4.5	bar ( g )

Temperature:	 	 156	C

Speed:			5600 	RPM

Steam flow

Process Extractions	- Flow			         120	ton/h

- Counterpressure  	94	ton/h

Generator

Capacity	31680	    KVA

Voltage	   13.8	    KV

Speed	  1800	    rpm

Estimated Replacement / Value US$:	To be Confirmed

Book Value: US$ To be Confirmed

Describe instruments and automation available:

This machine is controlled by a dedicated PLC ( Simadyn ) , and the MHI is a DCS from ABB which in the future will be converted to a Coros System , and the Protection Logic is configurated in an other PLC called Simatic .

There are also some Bentley Nevada Vibration Monitoring System and other pressure switch and interlocks to protect the whole machine against damage .