ENERGY EQUIPMENT DATA FOR PULP MILL

Steam Turbine / Generation

# Company: Votorantim Celulose e Papel   Date: 27/08/99   Responsible: Marcelo Castelli

#### Turbogenerator number  2  - ( TG – 2 )

### Steam Turbine

Manufacturer:	Siemens – Dedini

Serial number:	Impulse & Reaction Type Model 50/40

Year of Commercial Operation: 1999

## Capacity: 			38,600 kW

Number of operations hours: ~ 7000 equivalent hours

Counterpressure

## Pressure:			4.5	bar ( g )

Temperature:	 	 156	C

Speed:			3600 	RPM

Steam flow

Process Extractions	- Flow			  60	ton/h

- Conterpressure	192	ton/h

Generator

Capacity	48100	    KVA

Voltage	   13.8	    KV

Speed	  3600	    rpm

Estimated Replacement / Value US$:	To be Confirmed

Book Value: US$ To be Confirmed

Describe instruments and automation available:

This machine is controlled by a dedicated PLC ( Simadyn ) , and the MHI is a Coros System , and the Protection Logic is configurated in an other PLC called Simatic .

There are also some Bentley Nevada Vibration Monitoring System and other pressure switch and interlocks to protect the whole machine against damage.