# Summary of NOx Trading Programs

|  | Ozone Transport Commission (OTC) NOx Budget Program | Section 126 Federal NOx Budget Trading Program | NOx State Implementation Plan (SIP) Call |
| --- | --- | --- | --- |
| States Covered | CT, DC*, DE, MA, MD, ME, NH, NJ, NY, PA, RI, VT* *not in trading program | DC, DE*, IN, KY, MD*, MI, NC, NJ*, NY*, OH, PA*, VA, WV *Section 126 superceded by NOx SIP call (in states with approved SIPs) | AL*, CT, DC, DE, GA *, IL, IN, KY, MA, MD, MI*, MO* NC, NH, NJ, NY, OH, PA, RI, SC, TN, VA, WV *fine grid portions of the state only |
| Compliance period | May 1 - September 30 of each year. | May 1 - September 30 of each year | May 1 - September 30 of each year (In 2004, the compliance period begins May 31 for those states just launching their SIPs, for the OTR states it is May 1.) |
| Compliance Year | 1999 – 2003 (to be superceded by NOx SIP Calls in 2003) | 2003 | 2003: CT, DE, MA, MD, NH, NJ, NY, PA, RI 2004: All Other States (Unless further delayed by court actions.) |
| Emissions Cap | 219,000 tons in 1999; 143,000 tons in 2003 | 24, 905 tons (Non-EGU Budget) |  |
| Baseline Year | 1990 | 1995 | 1995 |
| Program Owner | OTC; Allowances set by OTC, program administered by EPA | EPA | States and EPA; States have the option of participating in the trading program and establishing unit allocations, program administered by EPA. States will be “FIP’d” (Federal Implementation Plan) if they don’t submit a SIP. To date, OH is the only state that has not made significant progress on their SIP. |
| Affected Units | All fossil fuel fired boilers or indirect heat exchangers with a maximum rated heat input capacity of 250 MMBtu/hour or more; and all electric generating facilities with a rated output of 15 MW or more. See link for state by state list of affected sources: http://www.epa.gov/airmarkets/otc/sources/index.html | The reality is that the only affected units will be the Non Electric Generating Units in the Non OTR states. See Appendix B for list of affected sources | Electric Generating Units and Non Electric Generating Units. Many Northeastern states have included EGUs >15MWs in their trading programs. Cogen Requirements still unclear: currently remanded to USEPA for further analysis. |