SUMMARY OF TEST RESULTS

(Date referenced is date tested unless not specifically indicated in test report)

# Loadport

27 June 2000	Auto-sample taken at loading by SGS Australia and sent to SGS Singapore for testing. “Ashing” test used for metals. All on spec.

Thailand

6 July 2000	Ship Composite sample taken before discharge in Thailand was tested by SGS Thailand for Carbon Residue 10% and Filterable Dirt.

- Filterable Dirt was off spec at 6.3;

- Carbon Residue 10% was off spec at 1.5.

SGS Thailand suggested that the Carbon Residue 10% result was questionable and recommended retesting. There were however, no other ship composite samples taken before discharge in Thailand. Shoretank samples tested by SGS Thailand for Carbon Residue 10% and Filterable Dirt on the same date were on spec.

Note : D5452 was used instead of D2276 for Filterable Dirt.

6 July 2000	Thailand shoretank samples taken every 2 metres and composite and tested by SGS Thailand for Filterable Dirt and Carbon Residue 10% varying results but all tested on spec.

12 July 2000	Thailand shoretank samples brought to SGS Singapore tested on spec except for Lead which tested off spec at 0.3 (T-961) and 0.2 (T-964). "Ashing" test used for metals.

13 July 2000 	Thailand shoretank samples brought to Caleb Brett Singapore all tested on spec, but note different test method used for metals (D5184/5185).

14 July 2000 	Thailand shoretank samples re-tested for metals by SGS Singapore:

Direct D3605

Vanadium off spec at 0.9 (T-964)

Lead off spec at 0.8 (T-961); 1 (T-9640;

Vanadium + Lead off spec at 1.2 (T-961); 1.9 (T-964);

Calcium on spec.

"Ashing"

All on spec.

18 July 2000	Thailand shoretank samples re-tested for Vanadium and Lead by SGS Singapore by Direct D3605 :

- Vanadium on spec;

- Vanadium + Lead on spec;

- Lead off spec at 0.2 (T-961); 0.3 (T-964).

Philippines

4 – 5 July 2000	Ship composite sample taken in Philippines tested by SGS Subic Bay (1st Test).

- Carbon Residue 10% was off spec at 1.8 (max 1.0);

- Water and Sediment was off spec at 0.15 (max 0.1); and

- Filterable Dirt was off spec at 120 (max 4.0).

Metals using Direct D3605 without “Ashing”:-

- Sodium + Potassium was off spec at 2.0 (max 1.0);

- Calcium was off spec at 2.7 (max 2.0);

- Lead was off spec at 1.2 (max 0.1); and

- Vanadium + Lead was off spec at 1.2 (max 0.5).

Note : Cargo was rejected by FGH based on this test result.

8-9 July 2000	Individual ship’s tanks samples (Upper/Middle/Bottom) re-tested by SGS Subic Bay (2nd Test)

- Carbon Residue 10%, varying results ranging from 0.6 (on spec) to 1.5 (off spec) (max 1.0);

- Water and Sediment, varying results ranging from <0.05 (on spec) to 0.5 (off spec) (max 0.1);

- Filterable Dirt, varying results ranging from 2.8 (on spec) to 133 (off spec) (max 4.0).

Metals using Direct D3605:-

- Sodium + Potassium, varying results off spec ranging from 1.1 to 2.8 (1.0 max);

- Calcium, varying results ranging from 1.3 (on spec) to 2.5 (off spec) (max 2.0);

- Lead, varying results off spec ranging from 1.0 to 2.5 (max 0.1); and

- Vanadium + Lead, varying results off spec ranging from 1.0 to 2.6 (max 0.5).

Metals using “Ashing” method (also labelled as AAS/Ital-3) : only Calcium was off spec ranging from 3.0 to 3.3 (max 2.0).

12-13 July 2000	Individual ship’s tanks samples (Running) re-tested by SGS Subic Bay (3rd Test)

- Carbon Residue 10% - varying results ranging from 0.6 (on spec) to 3.1 (off spec) (max 1.0);

- Filterable Dirt, varying results all off spec ranging from 85.2 to 121 (max 4.0).

Metals using Direct D3605:-

- Sodium + Potassium, varying results all off spec ranging from 1.3 to 2.7 (1.0 max);

- Calcium, varying results all off spec ranging from 2.1 to 2.5 (max 2.0);

- Lead, varying results all off spec ranging from 1.1 to 1.8 (max 0.1); and

- Vanadium + Lead, varying results all off spec ranging from 1.1 to 1.9 (max 0.5).

Metals using “Ashing” method (also labelled as AAS/Ital-3) all on spec except for Calcium ranging from 2.8 to 3.1 (max 2.0).

13-15 July 2000	Individual ship’s tanks samples (Comp; U/M/B) re-tested by SGS Subic Bay (4th Test)

- Carbon Residue 10% varying results but all on spec.

-	Filterable Dirt, varying results all off spec ranging from 46.2 to 178.3 (max 4.0);

-	Metals had varying results but all on spec for both ASTM D3605 Direct and “Ashing”

16 July 2000	Individual ship’s tanks samples (Comp; U/M/B) tested by ITS Caleb Brett Subic Bay.

- Carbon Residue 10%, varying results ranging from 0.4 (on spec) to 2.4 (off spec) (max 1.0);

- Filterable Dirt, varying results all off spec ranging from 13.7 to 623.5 (max 4.0);

Metals using “AAS” method:

- Sodium + Potassium, varying results ranging from 0.32 (on spec) to 2.09 (off spec) (1.0 max);

- Calcium, varying results ranging from 0.8 (on spec) to 2.8 (off spec) (max 2.0);

- Vanadium + Lead, varying results ranging from 0.12 (on spec) to 0.92 (off spec) (max 0.5);

- Lead varying results ranging from 0.01 (on spec) to 0.15 (off spec) (max 0.1);