- Application DSOW

TRANSMISSION ACCESS CHARGES OVERVIEW

## New Charge # 0372: High Voltage Access Charge due ISO

## New Charge # 0374: High Voltage Access Revenue due PTO

## New Charge # 0382: High Voltage Wheeling Charge due ISO

New Charge # 0383: Low Voltage Wheeling Charge due ISO

New Charge # 0384: High Voltage Wheeling revenue due TO

New Charge # 0385: Low Voltage Wheeling revenue due TO

Retired Charge # 0352: Wheeling Out/ Wheeling through due ISO

Retired Charge # 0354: Wheeling Charge refund due TO

## Background

At present, the three original Participating Transmission Owners (PTOs) namely: 1.Pacific Gas and Electricity (PG&E),  2.Southern California Edison (SCE)  and 3. San Diego Gas and Electricity (SDG&E), each files with the FERC to establish rates for their specific service area based on their individual Transmission Revenue Requirement (TRR) and their individual end-use load. The PTOs, through their Utility Distribution Companies (UDCs) will bill these rates to their end-use customers and wholesale customers.  (It should be noted that usually the PTO and UDC are business units within the same company).  ISO is not involved in this collection process.  However, the ISO does bill SCs for wheeling charges when they export energy from the ISO transmission grid to destinations outside the ISO Control Area or other non-participating UDCs.  The wheeling revenues collected by ISO are paid to the PTOs based on the ratio of their TRRs.

Upon California’s restructuring legislation (AB 1890) ISO became obligated to submit a revised Access Charge methodology to FERC.  Following an extensive  Stakeholders process  the ISO drafted the tariff language that was approved by the ISO Board on March 31, 2000 and filed with FERC as Amendment No. 27 to the ISO Tariff.  Amendment 27 presents the methodology for determining High Voltage Access Charge (HVAC), through which the embedded costs of the transmission facilities comprising the ISO Controlled Grid would be recovered.  The Access Charge methodology would also include a provision that mitigates cost shifts among the current and the New Participating TOs.

### Transmission Access Charge Methodology

The goal of the new Transmission Access Charge (TAC) methodology is to reach a uniform transmission access rate over the entire ISO service territory over a transition period of 10 years.  Under this new scheme, upon joining a new PTO to the ISO Control Area,  ISO will begin to collect a new High Voltage Access Charge (HVAC).  The existing Wheeling Access Charge and its disbursement mechanism will also be modified.  During the transition period, ISO’s Control Area will be divided into a number of “TAC Areas”.  Initially, there will be three TAC Areas, each of which corresponds to the original Control Areas of the existing PTOs.

- a Northern Area consisting of original Control Area of Pacific Gas and Electric Company.

- an East Central Area consisting of the original Control Area of Southern California Edison Company.

- a Southern Area consisting of the original Control Area of San Diego Gas & Electric Company.

- If the Los Angeles Department of Water and Power joins the ISO and becomes a PTO, its service area will form a fourth TAC Area, the West Central Area.

- If other utilities join ISO as PTOs, their service areas will either be merged with an existing TAC Area or a new TAC Area can be formed.

A new PTOs can provide the ISO with a notice of intent to join and execute the Transmission Control Agreement (TCA) by either January 1 or July 1 of any year.  Its service area will be incorporated into an ISO TAC area on those dates.

### High Voltage Access Charge (HVAC)

ISO will collect HVAC and Transition Charge from the UDCs/MSS based on the delivery of energy to Loads (including any distribution losses) on their systems.  For Loads that are not located on the system of an UDC, the HVAC and Transition Charge will be collected from the SC representing the Load.

The billing determinant for HVAC is the actual measured load (which includes any distribution losses).  These load measurements are reported to ISO through its Meter Data System.    Loads that are served by Existing Transmission Contracts are exempted from HVAC and Transition Charge.  Loads in a non participating UDCs area that are already subject to Wheeling Access Charge do not pay HVAC and Transition Charge.

Each TAC Area has its own HVAC rate which is applied to all the High Voltage Loads in the area.

Local Access Charge (below 200 kV) remains utility-specific and its collection will be the UDC’s responsibility.

HVAC ultimately will be based on one ISO Grid-wide rate.  Initially, the HVAC will be based on TAC Areas, which will transition 10% per year to the ISO Grid-wide.  In the first year after the Transition Date, the HVAC will be a blend based on 10% ISO Grid-wide and 90% TAC Area.  All new capital additions to existing high voltage facilities and new high voltage facilities will be included immediately in the ISO Grid-wide component.

- Collection of HVAC

HVAC and Transition Charge will be levied monthly by the ISO to the UDCs/MSS on the amount of energy taken off the ISO Control Grid to serve its load.  For loads that are connected directly to the ISO grid, HVAC and Transition Charge will be charged monthly to the SC representing the load. There will be an adjustment at the end of the month to reflect the ETC load that should be exempted from TAC charges.

#### Disbursement of HVAC Revenues

The ISO will collect and pay monthly to PTOs all HVAC and Transition Charge revenues at the same time as other ISO charges and payments are settled according to defined procedures in the Transmission Access Charge Tariff.

##### Adjustment of HVAC Rates and Refund Process

HVAC rates are subject to adjustments due to future FERC rulings on revised TRR filings or the Transmission Revenue Balancing Account (TRBA).  HVAC rates will be adjusted effective January 1 and July 1 of each year to reflect:  (1) the addition of any New Participating TO during the preceding six months and (2) changes to the High Voltage Transmission Revenue Requirements of any Participating TO that were accepted by the FERC or the ISO during the preceding six months.  HVAC rates are based on the accepted TRRs of the PTOs. Each semi-annual adjustment shall be made as follows.  Using the High Voltage Transmission Revenue Requirement approved by FERC or the ISO for each Participating TO during each month of the six months, the ISO will recalculate on a monthly basis the High Voltage Access Charge and Transition Charge applicable during such period.

### Wheeling Access Charge

Market Participants using the ISO Controlled Grid for the transmission of Energy to serve a Load located outside the ISO Controlled Grid pay the Wheeling Access Charge (WAC).  Holders of Existing Transmission Contracts (ETCs), when exercising their Existing Rights to transmit energy through the ISO grid, are exempted from WAC.  Currently, the ISO bills Scheduling Coordinators for energy that is wheeled through or wheeled out at Scheduling Points that exit the ISO Controlled Grid.  WAC revenues are collected by the ISO and then spread among the PTOs.

WAC revenues are treated as revenue credits to the PTOs Transmission Revenue Requirements, thereby reducing their net revenue requirements.

#### Determination of WAC Rates

##### WAC Rates for Single Owner Facilities

WAC rate will be defined for each Scheduling Point where energy may exit from the ISO grid into non-PTO owned facilities.  For a Scheduling Point that has a single owner, its High Voltage (HV) WAC rate will be equal to the HVAC rate of the area in which the owner resides.  If the Scheduling Point is at a Low Voltage (LV) level, in addition to HV WAC, a  LV WAC charge will also be assessed.  The LV WAC is based on the owner’s specific LV rate. WAC Rates for Joint Facilities

To the extent that more than one PTO owns or has firm entitlement to transmission capacity, less all Encumbrances at a Scheduling Point exiting the ISO Controlled Grid, the WAC rate reflect an average of the WAC rates applicable to those PTOs, weighted by the relative shares of their ownership or Entitlement to transmission capacity, less all Encumbrances at such Scheduling Point.  If the Scheduling Point is located at High Voltage Transmission Facilities, the WAC rate will consist of a High Voltage WAC component.  Additionally, if the Scheduling Point is located at Low Voltage Transmission Facilities, the applicable Low Voltage Wheeling Access Charge component will also be added to the WAC rate.

#### Disbursement of Wheeling Revenues

The ISO will collect and pay to PTOs all WAC revenues at the same time as other ISO charges and payments are settled.  WAC revenues collected at each Scheduling Point will be separated into two logical accounts: one for the High Voltage component and one for the Low Voltage component.  The amount in each account will be disbursed by the ISO to PTOs as described below.

##### Scheduling Point with owner PTOs in the Same TAC Area

With respect to revenues received for the payment of High Voltage component of WAC at a Scheduling Point at which all of the facilities and Entitlements, less all Encumbrances are owned by PTOs in the same TAC Area, the revenues will be disbursed to each such Participating TO based on the ratio of each PTO's High Voltage Transmission Revenue Requirement  to the sum of all such PTO's High Voltage TRRs.  If the Scheduling Point is located at a Low Voltage Transmission Facility, revenues received with respect to the Low Voltage component of the WAC will be disbursed to the PTOs  for wheeling to that Scheduling Point in proportion to their Low Voltage TRRs.

##### Scheduling Point without All Participating TOs in the Same TAC Area:

With respect to revenues received for the payment of Wheeling Access Charges for Wheeling to a Scheduling Point at which the transmission facilities and Entitlements, less all Encumbrances, are owned by Participating TOs in different TAC Areas, Wheeling revenues shall be disbursed to such Participating TOs as follows.  First, the revenues shall be allocated between such TAC Areas in proportion to the ownership and Entitlements of transmission capacity, less all Encumbrances, at the Scheduling Point of the Participating TOs in each such TAC Area.  Second, the revenues thus allocated to each TAC Area shall be disbursed among the Participating TOs in the TAC Area in accordance with the above  paragraph for Scheduling Point with owner PTOs in the Same TAC Area.

#### References

For further details on HVAC, Transition Charge, and Wheeling Billing & Settlement please refer to:

- ISO Tariff, Section 7

- ISO Tariff Appendix F, Schedule 3

- ISO Tariff Appendix F, F1-F2.1.1

- ISO Tariff Appendix H

- SABP 1.3.1k

Please note that Amendment 27 is currently being discussed in FERC proceedings (Docket # ER00-2019).  Therefore, This process is subject to future modifications.