# TECHNICAL CORNER ARTICLE

### A New Load Forecasting Function for Gas Sendout

## By Robert Lee, Hector Campos and Joe Hrgovcic

Gas sendout exhibits a primary dependence on temperature (or heating degree days) and has a characteristic “hockey stick” shape as shown in Fig. 1.  The flat region above 60 F represents the base load.  As temperature drops and heating load increases, there is a transition region to a linear increase in load with falling temperature.  Theoretically, a system reaches a maximum delivery capability at very low temperatures and the curve again flattens out.  Generally, the flattening is not apparent in weekly or quarterly data, but it does appear in the daily load data1 shown in Fig. 1.

This curve is frequently represented analytically by piecewise linear regressions over the flat, transition and linear portions of the curve.  Other variables such as wind speed, previous day’s load, day of week, and average temperature over several days are included to improve the fit.  Some disadvantages of this modeling approach are the large number of para-meters and the discontinuities in load prediction at the boundaries of the linear regression regions.

Previous attempts to fit this curve shape with a complex function, over the full temper-ature range, such as the logistic function (which has a characteristic S shape), have not been successful.  Here, we have applied a complex function from option pricing theory (a call spread) to get a good fit over the full data range.  This function is shown in Fig 2.

where

A

B

T1

C1

C2

T2

Load

Tavg

Figure 2 New Fitting Function

N(.) is the cumulative

normal distribution

In addition to providing a superior fit in this case, this function also has an intuitive appeal.  The five parameters have the interpretation:

A – maximum load capability

B – “distance” between base and maximum loads

T1 – temperature at which the curve flattens at low temperatures

C1 – curvature or rate at which it flattens

T2 – temperature at which the curve flattens at high temperatures

C2 – curvature or rate of flattening to base load

The result of using this function to fit the data of Fig. 1 is shown in Fig. 3.

The parameter values for this curve are:

# A = 2,237,280

# B = 53,862

# T1 = 31.47

# C1 = 0.52

# T2 = 60.53

C2 = 0.11

The parameters were opti-mized using Excel Solver.  The result is very sensitive to the initial guess used in Solver.  A good initial guess can be established by graphing the data and function in Excel, and adjusting the parameters to give a good visual fit.

Additional flexibility can be obtained with a sixth parameter that allows different notional amounts of V(T1,C1) and V(T2,C2).  In the example, an R2 = 0.95 was obtained with five parameters; the sixth parameter did not show a significant improvement.   A piecewise linear fit had R2 = 0.90 to 0.92 in the different segments.

In situations where this function produces a good fit to load data, a simple look-up table can then be produced for use by traders.

1The data for this project was provided by Goeff Storey and Sean Boyle.