- TRANSWESTERN PIPELINE COMPANY

- BUSINESS PLAN

- Insert one page system map for cover page (communications to provide)

- Page 2

- Overview

- Transwestern owns and operates an interstate natural gas pipeline engaged in the transportation of natural gas.  Through its approximately 2,500-mile pipeline system, Transwestern transports natural gas from West Texas, Oklahoma, eastern New Mexico and the San Juan Basin in northwestern New Mexico and southern Colorado primarily to the California market and to markets off the east end of its system.  Transwestern has access to three significant supply basins for its gas supply: the San Juan Basin, the Permian Basin in West Texas and eastern New Mexico and the Anadarko Basin in the Texas and Oklahoma Panhandles.   Additionally, gas from the Rocky Mountain Basin can access Transwestern through pipeline interconnections. Transwestern's peak delivery capacity was approximately 1.7 billion cubic feet per day in 2001.  .  Transwestern Pipeline Company is an indirect wholly-owned subsidiary of Enron Transportation Services Company (“ETS”), which is a majority owned subsidiary of Enron Corp. The complete organizational chart of Tranwestern Pipeline is displayed below.

# History

Transwestern was constructed in 1960 to deliver natural gas to Southern California Gas Company (“SoCalGas”), which is the local distribution company for Los Angeles, California.  Transwestern serves San Diego and Long Beach, California as well as surrounding communities via SoCalGas.  Transwestern’s system, comprised of the mainline, West Texas lateral and Panhandle lateral, was constructed in several phases from 1960 to 1982, with capacity increasing from an original 350 MMcf/d to 750 MMcf/d.

In 1991, Transwestern completed a major expansion of its mainline, construction of its San Juan lateral (from the four-corners area of New Mexico to Transwestern’s mainline at Thoreau) and construction of its Topock lateral (from its mainline in Arizona to the California border at Topock).  These new laterals gave Transwestern critical access to the burgeoning San Juan supply basin and to incremental Northern California markets via Pacific Gas & Electric (“PG&E”), which is the local distribution company for San Francisco, California.,  With this expansion, Transwestern also began to serve Arizona markets for the first time.

Transwestern has continued to expand its facilities in response to increasing demand in the California and Arizona markets as well as demand for access to economic supplies.  In 1996, Transwestern purchased a 72% ownership interest from Northwest Pipeline Company in 33 miles of pipe, extending its San Juan lateral into La Plata County, Colorado.  Simultaneously, Transwestern expanded the original portion of its San Juan lateral by 255 MMcf/d to provide access to additional San Juan Basin supplies.  During this same time frame, deliverability to mid-continent interstate and Texas intrastate interconnects on the East-end of Transwestern’s system were in high demand.  Transwestern responded to this market signal by modifying its sytem to allow bi-directional capability on its mainline to flow San Juan gas East from Thoreau.  Transwestern will be placing its Red Rock Expansion in-service June, 2002 bringing total capacity on the mainline to 1,210 MMcf/d.  Current capacity on the San Juan lateral is 850 MMcf/d.

Key Value Considerations

- Strong Demand Growth

- Stable, Identifiable Customer Base

- Flexible System

Via Transwestern's San Juan lateral pipeline, the San Juan Basin gas may be delivered to California markets as well as markets off the east end of Transwestern's system.  This bi-directional flow capability was added in 1995 to increase system flexibility and utilization.

- Market and Supply Overview

- Market

Transwestern’s pipeline was built primarily to transport gas from the gas supply basins in the Southwest United States to the growing California markets.  According to the California Energy Commission, California’s natural gas usage will increase from 6,400 MMcfd in 2000 to 7,500 MMcfd in 2010 with a 2.5% annual average demand growth for electric generation.

## Insert Forecasted Natural Gas Demand graph

As indicated in the following chart, the power generation markets for natural gas in Arizona, New Mexico and Nevada are expected to grow from 24.2 GW with 24% natural gas usage to 36.0 GW with 49% natural gas usage.

Insert pie chart indicating growth and fuel use.

California, while consisting of a number of quite different micro-climates, overall has a mild climate.  If you combine the cooling and heating degree-days, California ranks last.

|  | Population per Year 2000 Census |  |  |  | Degree Days in 1999 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Count | Rank | Decade Growth Rates |  | Cooling |  | Heating |  |
|  |  |  | Past | Projected | Count | Rank | Count | Rank |
| California | 33,871,648 | 1 | 0.87% | 1.48% | 756 | 31 | 2600 | 40 |
| Nation |  |  | 0.99% | 0.81% | 1297 |  | 4169 |  |

This is one big reason why their per capita energy consumption is one of the lowest in the nation.  This is offset by their being number one in population which makes them number one in residential, commercial and transportation energy consumption and number two overall.  California, in addition to being number one in population has been experiencing

Reasonable growth bit is projected to grow much faster in the first decade of the new millennium.  Power sales have grown considerably.  There are some quirks in the data which may be a result of mischaracterizing some of the commercial and industrial sales.

Residential power sales have been growing about 1.5% per year.  Commercial and industrial have grown about 2.2 to 2.7% per year, which is about twice as fast as the population has grown.  Because of the dysfunctional nature of the California power market the past couple of years, recent growth might be abherrant.

As was the case in Florida, gas-fired power generation has increased rapidly.  Oil-fired generation has virtually disappeared since about 1991.  The 2000 figure for imports is actually an aberration.  In 1999, imports were virtually the same 1995.  This is one of the dangers of depending on imports from other states which may have their own problems.

|  | Generation, M Gigawatt-Hrs |  |  | Growth | Percentage Share of Generation |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1990 | 1995 | 2000 | Rate/Yr | 1990 | 1995 | 2000 |
| Hydro | 26,092 | 51,665 | 42,053 | 4.89% | 14% | 25% | 17% |
| Nuclear | 36,586 | 36,186 | 43,533 | 1.75% | 19% | 17% | 17% |
| Coal | 21,402 | 17,925 | 36,804 | 5.57% | 11% | 9% | 15% |
| Gas | 76,082 | 78,378 | 106,878 | 3.46% | 40% | 38% | 42% |
| Geothml. | 16,038 | 14,267 | 13,456 | -1.74% | 8% | 7% | 5% |
| Other | 16,038 | 14,267 | 13,456 | -2.52% | 7% | 5% | 4% |
| Imports | 61,959 | 47,514 | 30,410 | -6.87% | N.A. | N.A. | N.A. |

In addition to being heavily dependent on imports and hydro, California has become heavily dependent on non-utility generation.  In 1985, only about 6% of California’s in-state power generation came from non-utility generators.  In 2000, almost 50% of the total and almost 90% of gas-fired was non-utility.

|  |  |  | % Non-Utility Generation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1985 | 1990 | 1995 | 2000 |
|  |  | Total | 6% | 27% | 29% | 47% |
|  |  | Gas-Fired | 9% | 41% | 50% | 87% |

Up until about 1996, the power generation market for gas and as a result the entire gas market was very stable.  Since then, the market has grown rapidly, primarily as a result of increased gas-fired generation, mostly non-utility.  (In the figure below, the industrial volumes reported by the Dept. of Energy (DOE) have been split into traditional industrial and non-utility generation categories using generation information from the California Energy Commission and  DOE consumption data from 1999 and 2000.)

From 1996 thru 2000, residential gas sales have grown over 3.5% per year and commercial/industrial by almost 6% per year. Power generation grew by almost 7% per year, even when ignoring the dysfunctional power year 2000.  Just as was the case with power sales, there appears to be some misclassification of commercial and industrial sectors which necessitates combining the two for analysis.

# Supply

Transwestern's mainline includes a lateral pipeline to the San Juan Basin which allows Transwestern to access San Juan Basin gas supplies.  Via this San Juan lateral pipeline, the San Juan Basin gas may be delivered to California markets as well as to markets off the east end of Transwestern's system.  As noted below, the San Juan basin production peaked in 1999 at 4.3 Bcfd with an expected decline of 2.4% to 3.5 Bcfd in 2010.

## Insert San Juan Basin Forecast graph

In addition to its San Juan basin gas supply, Transwestern has positioned itself to supplement this with gas supply from the Rocky Mountain region via its Northwest Pipeline and TransColorado interconnects.  As indicated by the following map, the shift of the Canadian gas supplies from Northern California to the Mid-Continent region has resulted in increased Permian and Anadarko gas supplies via Transwestern’s interconnects east of Thoreau, New Mexico.

## Insert Permian and Anadarko Supplies Shift map

Transwestern has firm transportation service on the east end of its system and transports Permian, Anadarko and San Juan Basin supplies into Texas, Oklahoma and the Midwestern United States.  More recently, Transwestern has modified its operations to enhance its ability to supply gas to the California market by completing expansions in 2000 and 2001, increasing its delivery capacity to California by 140 MMcfd, and 50 MMcfd, respectively.  Transwestern is nearing completion on yet another expansion of 150 MMcfd with expected in-service date of July 2002 (Red Rock Expansion).

# Competitive Threats

Transwestern competes with several interstate pipelines in the California market and its markets off the east end of its system.  As indicated below, Transwestern’s maintains approximately 15% of the available pipeline capacity to the California markets.

## Insert Current Interstate Pipelines to California slide

In addition, several proposed expansions to California are in advanced stages of development as shown below.

## Insert Proposed Expansions to California slide

Customers and Contracts

As shown below, Transwestern benefits from a broad customer base.  Its capacity is held 38% by local distribution companies, 33% by marketing companies, 24% by producers and 5% by end-users.

Insert Top 10 Customers by Revenue in 2001 slide

Transwestern enjoys a collaborative relationship with its customers.  In 1995 Transwestern reached a landmark settlement with its customers to address the issue of stranded capacity costs associated with turnback of SoCalGas capacity.  Transwestern’s settlement was lauded for its proactive, collaborative approach to problem resolution and was held as an example for El Paso’s subsequent settlement in resolution of their stranded cost issues.

Historically, approximately 90% of the volumes scheduled on Transwestern have been on a firm transportation basis and into the future Transwestern maintains a significant portion of its capacity under firm contract with its customers.  Currently, Transwestern’s capacity for both west and east flow is fully subscribed under a combination of short and long-term contracts.  Relatively small increments of operational capacity become available from time to time and are generally sold on a daily or short-term basis.

## Insert Capacity Subscription Level By Segment slide

Under a firm transportation service agreement, the customer “owns” its capacity on Transwestern’s system.  Transwestern reserves such capacity for the customer as well as guaranteeing delivery of gas scheduled under the agreement.  In exchange for this reservation and guarantee of service, the customer pays Transwestern a demand charge for such capacity whether or not it is utilized.  In 2002, Transwestern’s margins are projected to be 80% from demand/fixed sources and 20% from commodity/variable sources.

## Insert Gross Margin by Type slide

Though Transwestern is guaranteed demand revenues regardless of gas flow, Transwestern continues to maximize the utilization of its pipeline system to increase firm  commodity revenue, interruptible transportation revenue and operational efficiencies.

## Insert Load Factors & Throughput Growth slide

Ownership Structure

Source: Enron Corp.

- The map below displays Transwestern’s Southwest pipeline facilities.

Transwestern Pipeline Map

Source: Enron Corp.

Transwestern's mainline includes a lateral pipeline to the San Juan Basin which allows Transwestern to access San Juan Basin gas supplies.  Via Transwestern's San Juan lateral pipeline, the San Juan Basin gas may be delivered to California markets as well as markets off the east end of Transwestern's system.  This bi-directional flow capability enhances pipeline utilization.  Transwestern added bi-directional flow capability in 1995 to increase system flexibility and utilization.

Transwestern has firm transportation service on the east end of its system and transports Permian, Anadarko and San Juan Basin supplies into Texas, Oklahoma and the midwestern United States.  More recently, Transwestern has modified its operations to enhance its ability to supply the California market.  In May 2000, Transwestern completed its Gallup Expansion project which increased delivery capability to California by 140 million cubic feet per day.  In June, 2002, Transwestern will place its Red Rock Expansion in-service, increasing its mainline capacity by 120 MMcf/d to 1,210 MMcf/d.

[SEE CHART – WHAT IS THE UNIT OF MEASURE FOR VOLUMES?]

($ in millions)

| Top 10 Customers |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rank | Customer | Sales | % Total Sales | Volumes | % of Total Volumes | Ratings | Contract Maturity |
| 1 | Southern California Gas Co. | $56.3 | 31.2% | 176.8 | 19.1% | AA- | 10/31/05 |
| 2 | Texaco Natural Gas | 16.2 | 8.9% | 34.4 | 3.7% | NR* | various to 07 |
| 3 | PG&E Energy Trading – Gas Corp. | 16.0 | 8.8% | 41.1 | 4.4% | BBB+* | various to 02 |
| 4 | Sempra Energy Trading Corp. | 14.6 | 8.1% | 59.6 | 6.4% | A* | various to 05 |
| 5 | BP Energy Company | 10.9 | 6.0% | 68.5 | 7.4% | AA+* | various to 08 |
| 6 | Duke Energy Trading and Marketing | 7.9 | 4.4% | 46.1 | 5.0% | A- | various to 07 |
| 7 | El Paso Energy Marketing Company | 7.3 | 4.0% | 30.8 | 3.3% | BBB+* | various to 07 |
| 8 | Burlington Resources Trading | 5.2 | 2.9% | 40.7 | 4.4% | A-* | various to 06 |
| 9 | Agave Energy Co. | 4.6 | 2.6% | 42.7 | 4.6% | NR | various to 03 |
| 10 | U S Gas Transportation | 4.4 | 2.4% | 59.2 | 6.4% | NR | 03/31/03 |
|  | Totals | $143.3 | 79.4% | 599.8 | 64.8% |  |  |

Note: * indicates rating at parent company level

- The following table sets forth selected financial data for Transwestern for fiscal years 1998 through estimated 2001.

($ in millions)

| Summary Financial Data |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1998 | 1999 | 2000 | 2001E |
| Revenue | $167.6 | $155.7 | $176.8 | $198.2 |
| % growth | NA | (7.1)% | 13.6% | 12.1% |
| EBITDA | 123.0 | 106.6 | 126.0 | 138.2 |
| % margin | 73.4% | 68.4% | 71.3% | 69.7% |
| Interest Expense | $12.8 | $11.7 | $11.1 | $26.7 |
| Capital Expenditures | 18.7 | 34.2 | 29.6 | 65.7 |
| Total Debt | 146.1 | 142.3 | 165.4 | 553.1 |
| EBITDA/Interest Expense | 9.6x | 9.1x | 11.4x | 5.2x |
| (EBITDA-CAPEX)/Interest | 8.2x | 6.2x | 8.7x | 2.7x |
| Total Debt/EBITDA | 2.0x | 1.3x | 1.3x | 4.0x |

According to the Lehman Report by Ted Izatt and J. Robert Chambers, Izatt and Chambers have performed a valuation analysis on Transwestern.  The following chart summarizes the valuation analysis.

- ($ in millions)

| Summary Valuation |  |
| --- | --- |
| Revenue | $207.0 |
| EBITDA | 131.0 |
| EBITDA Multiple | 8.5x |
| Total Enterprise Value | $1,112.0 |
| Less Operating Company Debt | 562.0 |
| Holding Company Value | $551. |
| Less Holding Company Debt/Preferred Stock | 0.0 |
| Holding Company Equity Value | 551.0 |
| Enron Net Interest | 100% |
| Enron Equity Value | $551.0 |