## C.  TranSportation Tolling Rate

ANGTS’s toll model calculates a cost of service transportation tolling rate over an operating period of 25 years.  The following two cases were analyzed:

| Base: | 4,344 MMcf/d delivered (4,500 MMcf/d receipt) in year one and remaining constant through year 25. |
| --- | --- |
| Expansion: | 4,344 MMcf/d delivered (4,500 MMcf/d receipt) in year one, growing to 5,122 MMcf/d delivered (5,400 MMcf/d receipt) in year three and remaining constant through year 25. |

Detailed capital costs for these cases can be found in the Engineering Design and Capital Cost section, Schedule 2.  The two cases described above were modeled using the following common assumptions:

Tax Assumptions:

- US Tax Rate: Combined legislated  tax rate 41.1% (Federal:  35.0%, Alaska:  9.4%)

- US Property Taxes:  2%

- US Tax Calculation Method: Normalized

- Canadian Tax Rate:  Legislated Tax Rate, combined Federal/Provincial Rate (Yukon 36.0%, BC 34.5%, Alberta 34.5%)

- Canadian Property Taxes:  1%

- Canadian Tax Calculation Method: Flowthrough

Tax Depreciation:

- US:  15 Year MACRS

- Canada:  4% for pipeline and 20% for compression

Capital Structure:

- 70% Debt / 30% Equity

- Interest Rate:  7.5%

- Return on Equity:  14%

- Weighted Average Cost of Capital:  9.45%

- Working Capital: 0.5% of Capex

- Project life: 25 years in operation; Terminal Value of 20% of installed cost

- Annual Depreciation Rate: 3.2%

Operating Expenses:

- Indirect O&M costs per year:  1% of Capex

Cost Escalation:

- Capex Escalation Rate per year:  2.5%

- Opex Escalation Rate per year:  2.5%

Fuel:

- Base: 3.7% of receipt volume of 4,500 MMcf/d

- Expansion: 5.1% of receipt volume of 5,400 MMcf/d

Term of Transportation Contract:

- 25 year, renewable

- In the event of non-renewal, balance of residual depreciation recovered during last five years of contract

Other Project Data:

- First Gas:  Nov, 2007

- Start-up Date:  July, 2008 (full rate flow)

- Canadian Currency Exchange Rate: 1 USD = 1.5 CAD

- Load Factor:  100%

- AFUDC calculated on blended weighted average cost of capital

- Linepack included at $2/Mcf

Indicative Tolls:

The above assumptions for the two cases outlined, stated on volumes delivered to Gordondale, yield the following indicative cost of service tolls, not including fuel.  These are stated per MMbtu in 2008 US dollars.

|  | Indicative Cost of Service Rate ($/MMbtu) |  |
| --- | --- | --- |
| Case: | Base 4,500 MMcf/d | Expansion 5,400 MMcf/d |
|  | Delivered Volume (MMcf/d): 4,344 MMcf/d | Delivered Volume (MMcf/d): 4,344 MMcf/d – 5,122 MMcf/d |
| Year 1 2008 | 1.43 | 1.43 |
| Year 5 2013 | 1.32 | 1.19 |
| Year 10 2018 | 1.16 | 1.09 |
| Year 15 2023 | 0.98 | 0.93 |
| Year 20 2028 | 0.84 | 0.80 |
| Year 25 2033 | 0.71 | 0.67 |

The above assumptions for the two cases outlined, stated on volumes delivered to Gordondale, yield the following indicative levelized tolls, not including fuel. These are stated per MMbtu in 2008 US dollars.

| Case | Indicative Levelized Rate ($/MMBtu) |
| --- | --- |
| Base 4,344 MMcf/d: | 1.17 |
| Expansion 5,122 MMcf/d: | 1.10 |

The indicative rates for both the costs of service and levelized methods are compared in the following chart.

Figure 2 – Project Life Toll

Toll Enhancements

The opportunity exists to improve upon the above tolls.  This can be accomplished by a collaborative approach of all stakeholders in the commercialization of Alaska gas.   These include the ANS Shippers, the ANGTS Group and governments.  The ANGTS Group has examined several possibilities to reduce the toll during the initial years of the project to achieve an acceptable target.  A partial list of enhancements is provided below, together with the estimated toll impact.

- Property tax concessions: holiday during construction and initial five years ( -$0.12 per MMbtu)

- Alaska State Income Tax: reduction ( -$0.01 per MMbtu)

- Government loan support: reduce cost of borrowing to 6% (-$0.11 per MMbtu)

- MACRS at 7 years ( -$0.01 per MMbtu)

By relying on a collaborative approach of stakeholders and employing a combination of available measures, the first-year cost of service toll for the base case (4,344 MMcf/d) can be reduced from $1.43 to the range of $1.20 to $1.25 per MMbtu.  If the ANGTS sponsors can rely on levelized tolling, these enhancements would be applicable to reducing the levelized rate.

Sensitivities:

A sensitivity analysis was conducted for the cost of service and the expansion cases.  The results are presented on the following charts:

Figure 3 – Base Case Toll Sensitivities