# Data Dictionary 							            Version 14.0

FILE ONE: Hourly Data:  MM-DD-YYYY Hour Vxx

- Power Suppliers Data

| POWER SUPPLIERS | Generator Information header |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Organization name | VARCHAR2(50) | ID | Name of the Organization |  |
| 200 | Gen name | VARCHAR2(50) | ID | Name for the Generator |  |
| 201 | Gen Ptid | NUMBER(5) | ID | NYISO assigned point identifier |  |
| 101 | Start day | DATE Format: MM/DD/YYYY | Date | Start Date |  |
| 102 | Start hour | NUMBER(2) | Hour | Start Hour |  |
| 202 | Day Ahead Hrly LBMP MWh | NUMBER(6,1) | MWh | LBMP MWs sold Day-Ahead, based on decremental bid may be (-) which results in a purchase of LBMP MWs Day-Ahead (Day-Ahead Scheduled MWs – Bilateral Transaction MWs) | + = Sale to the ISO |
| 203 | Day Ahead Hrly LBMP $ | NUMBER(15,2) | $$ | LBMP price at this generator bus |  |
| 204 | Day Ahead forward Energy $ | NUMBER(15,2) | $$ | Generator total DAM LBMP value | + = Due Generator |
| 205 | Day Ahead BPCG $ | NUMBER(15,2) | $$ | Generator bid production cost | + = Due Generator |
| 206 | Day Ahead Startup $ | NUMBER(15,2) | $$ | Start up payment | + = Due Generator |
| 207 | Hrly Integrated R/T Balancing MWh | NUMBER(6,1) | MWh | Time weighted hourly MWh value to be billed, can be calculated using the actual metered, SCD ramped base point or AGC ramped base point at each SCD interval as the net from day ahead to real time. | + = Sale to the ISO |
| 208 | Hrly Integrated R/T Bus LBMP $ | NUMBER(15,2) | $$ | Time weighted and load weighted hourly Real-Time LBMP price at this generator bus |  |
| 209 | Hrly Balancing Energy $ | NUMBER(15,2) | $$ | Generator total balancing LBMP value | + = Due Generator |
| 210 | R/T BPCG $ | NUMBER(15,2) | $$ | Bid production cost value for energy scheduled and delivered in the real time market not covered in the day ahead market | + = Due Generator |
| 211 | R/T Startup $ | NUMBER(15,2) | $$ | Start up cost value for start ups scheduled in the hour ahead or supplemental markets | + = Due Generator |
| 212 | Monthly Voltage Service $ (constant) | NUMBER(15,2) | $$ | Monthly generator voltage service rate for units providing FERC Form 1 data this value is the unit fixed yearly rate. For all others this value is the NYISO calculated voltage service rate. |  |
| 213 | % Time Unit or Synch Cond I/S (hour) | NUMBER | ## |  |  |
| 214 | Voltage Support $ | NUMBER(15,2) | $$ | Voltage payment based on in service time and fixed rate or fixed payment schedule for units under contract to supply installed capacity | + = Due Generator |
| 215 | Hrly VSS LOC $ | NUMBER(15,2) | $$ | Lost opportunity cost value for units directed to hold a MW output to support voltage | + = Due Generator |
| 216 | Availability Index | NUMBER | ## | % of number of seconds unit is on control for the hour |  |
| 217 | Hrly Day Ahead Reg Avail | NUMBER(6,1) | MWh | Regulation accepted Mws |  |
| 218 | Hrly Day Ahead Reg MC $ | NUMBER(15,2) | $$ | Regulation market clearing price |  |
| 219 | Hrly Suppl Reg Avail | NUMBER(6,1) | MWh | Regulation supplemental accepted availability MWs |  |
| 220 | Integrated Hrly Suppl MC $ | NUMBER(15,2) | $$ | Regulation supplemental market clearing price |  |
| 221 | Reg Replacement cost $ | NUMBER(15,2) | $$ | Regulation availability replacement charge | + = Due ISO |
| 222 | Regulation Charge $ | NUMBER(15,2) | $$ | Regulation charge for causing regulation | + = Due ISO |
| 223 | Hrly 30 Min MCP $ | NUMBER(15,2) | $$ | 30Min Reserve market clearing price |  |
| 224 | Hrly 30 Min Res MWHr | NUMBER(6,1) | MWh | 30Min Reserve accepted MWs |  |
| 225 | Hrly Suppl 30 Min MCP $ | NUMBER(15,2) | $$ | 30Min reserve supplemental market clearing price |  |
| 226 | Hrly Suppl 30 Min Res MWHr | NUMBER(6,1) | MWh | 30Min reserve accepted supplemental availability MWs |  |
| 227 | Hrly 30 Min Res Avail $ | NUMBER(15,2) | $$ | Total 30Min reserve payment | + = Due Generator |
| 228 | Hrly Synch Res MCP $ | NUMBER(15,2) | $$ | 10Min Synchronous reserve market clearing price |  |
| 229 | Hrly Synch Res MWHr | NUMBER(6,1) | MWh | 10Min Synchronous reserve accepted MWs |  |
| 230 | Hrly Suppl Synch Res MCP $ | NUMBER(15,2) | $$ | 10Min synchronous reserve supplemental market clearing price |  |
| 231 | Hrly Suppl Synch Res MWHr | NUMBER(6,1) | MWh | 10Min synchronous reserve supplemental accepted availability MWs |  |
| 232 | Hrly Synch Res Avail $ | NUMBER(15,2) | $$ | Total 10Min synchronous reserve payment | + = Due Generator |
| 233 | Hrly 10 Min Non Synch MCP $ | NUMBER(15,2) | $$ | 10Min Non-Synchronous reserve market clearing price |  |
| 234 | Hrly 10 Min Non Synch Res MWHr | NUMBER(6,1) | MWh | 10Min Non-Synchronous Reserve accepted MWs |  |
| 235 | Hrly Suppl 10 Min Non Synch MCP $ | NUMBER(15,2) | $$ | 10Min non-synchronous reserve supplemental market clearing price |  |
| 236 | Hrly Suppl 10 Min Non Synch MWHr | NUMBER(6,1) | MWh | 10Min non-synchronous reserve supplemental accepted availability MWs |  |
| 237 | Hrly 10 Min Non Synch Res Avail $ | NUMBER(15,2) | $$ | Total 10Min non-synchronous reserve payment | + = Due Generator |
| 238 | Hrly LRR DAM Contract Balancing Payment $ | NUMBER(15,2) | $$ | LRR payment to make units whole for being dispatched below their day-ahead schedule out-of-merit | + = Due Generator |
| 239 | Hrly DAM Contract Balancing Payment $ | NUMBER(15,2) | $$ | Payment to make units whole for being dispatched below their day-ahead schedule out-of-merit | + = Due Generator |
| 240 | Hrly Synch Res LOC $ | NUMBER(15,2) | $$ | Spinning reserve lost opportunity payment | + = Due Generator |
| 241 | Hrly 30 Min Res Reduction MWHr | NUMBER(6,1) | MWh |  |  |
| 242 | Synch Res Reduction MWHr | NUMBER(6,1) | MWh |  |  |
| 243 | Hrly 10 Min Non Synch Res Reduction MWHr | NUMBER(6,1) | MWh |  |  |
| 244 | Reserve Penalty $ | NUMBER(15,2) | $$ | Requested reserve shortfall charge | + = Due ISO |
| 245 | Avg Supply Ratio | NUMBER(15,4) | ## | Reserve supply Performance ratio |  |
| 246 | Hrly 10 Min Non Synch Res LOC $ | NUMBER(15,2) | $$ | 10Min non-synchronous reserve lost opportunity cost payment | + = Due Generator |
| 5200 | Out of Merit Flag | VARCHAR2(1) | ID | Y/N Out of Merit indication |  |
| 5210 | Local Reliability Flag | VARCHAR2(1) | ID | Y/N indication if unit out of merit for local reliability |  |
| 5220 | Out of Merit Flag Memo | VARCHAR2(200) | ID | Reason for unit placed out of merit |  |
| 5230 | Eligible Mingen Flag | VARCHAR2(1) | ID | Eligible for min gen payment Y/N flag |  |
| 5240 | Ramp rate constraint flag | VARCHAR2(1) | ID | Y/N indication if unit was ramp rate constrained |  |
| 5250 |  |  |  |  |  |
| 5260 | Regulating unit flag | VARCHAR2(1) | ID | Y/N indication of regulating units |  |
| 5270 | Dispatch Seconds | NUMBER(6) | ## | Number of seconds during the hour the unit was on dispatch. |  |
| 5280 | Number of Reserve Pickup Intervals | NUMBER(1) | ## | Number of reserve pick-ups during the hour |  |

End of row 1 (hour starting 0) of generator,  to be followed by hour starting 1-23, followed by next generator for this organization, and continuing to generator (n).

II.	Transmission Customers Data - LSE LBMP Energy

| LSE LBMP ENERGY | Header for LSE LBMP Information |
| --- | --- |

| Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID | Name of the Organization |  |
| 400 | LSE name | VARCHAR2(50) | ID | Name of Load Serving Entity(for External Transaction this will be NULL) |  |
| 401 | Load Bus name | VARCHAR2(50) | ID | Name of Load Bus |  |
| 101 | Start day | DATE Format: MON/DD/YYYY | Date | Start Date |  |
| 102 | Start hour | NUMBER(2) | Hour | Start Hour |  |
| 402 | Day Ahead Hrly LBMP MWh | NUMBER(6,1) | MWh | LBMP MWs purchased Day-Ahead, (Load bid MWs) | + = Purchase from the ISO |
| 403 | Day Ahead LBMP $, by zone | NUMBER(15,2) | $$ | Zone LBMP price where this load bus is located |  |
| 404 | Hrly Fwd Energy $ | NUMBER(15,2) | $$ | Energy component cost | + = Due ISO |
| 405 | Hrly Fwd Loss $ | NUMBER(15,2) | $$ | Loss component cost | + = Due ISO |
| 406 | Hrly Fwd Cong $ | NUMBER(15,2) | $$ | Congestion component cost | + = Due ISO |
| 407 | LSE Hrly Integrated R/T Balancing MWh by bus | NUMBER(6,1) | MWh | Time weighted hourly load bus estimate based on LSE forecast, adjusted for NYISO measured subzone load proportional with other LSE’s in this subzone, net from day ahead schedule and all bilateral transaction MWs | + = Purchase from the ISO |
| 408 | R/T LBMP $, by zone | NUMBER(15,2) | $$ | Time weighted and load weighted hourly Zonal LBMP price where this load bus is located |  |
| 409 | Hrly Balancing Energy $ | NUMBER(15,2) | $$ | Energy component cost | + = Due ISO |
| 410 | Hrly Balancing Loss $ | NUMBER(15,2) | $$ | Loss component cost | + = Due ISO |
| 411 | Hrly Balancing Cong $ | NUMBER(15,2) | $$ | Congestion component cost | + = Due ISO |

End of row 1 (hour starting 0)  for Organization / LSE 1/Zone 1 energy  data, to be followed by hour starting 1-23, followed by next LSE for this organization, and continuing to LSE (n)/Zone(n)

III.	Transmission Customers Data    - Transaction TUC and TSC Charges

| TRANSACTIONS | Header for Transaction Information |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID | Name of the Organization |  |
| 400 | LSE name | VARCHAR2(50) | ID | Name of Load Serving Entity(for External Transaction this will be NULL) for the transaction sink |  |
| 500 | Trans_id | NUMBER | ID # | Unique transaction Identifier |  |
| 101 | Start day | DATE Format: MON/DD/YYYY | Date | Start Date |  |
| 102 | Start hour | NUMBER(2) | Hour | Start Hour |  |
| 501 | Day Ahead Scheduled Transactions | NUMBER(6,1) | MWh | Day ahead transaction MWh amount | + = Bilateral Scheduled |
| 502 | Hrly Transaction Day Ahead Loss $ | NUMBER(15,2) | $$ | Transaction Loss component cost | + = Due ISO |
| 503 | Hrly Transaction Day Ahead Congestion $ | NUMBER(15,2) | $$ | Transaction Congestion component cost | + = Due ISO |
| 504 | Hrly Day Ahead TUC $ | NUMBER(15,2) | $$ | DAM Transmission use charge (loss_cost + Cong_cost) | + = Due ISO |
| 505 | R/T Scheduled Transactions | NUMBER(6,1) | MWh | R/T transaction MWhs | Negative = Bilateral Curtailed |
| 506 | R/T Balancing Loss $ | NUMBER(15,2) | $$ | Cost of losses on transaction | + = Due ISO |
| 507 | R/T Balancing Congestion $ | NUMBER(15,2) | $$ | Cost of congestions on transaction | + = Due ISO |
| 508 | Hrly R/T TUC $ | NUMBER(15,2) | $$ | R/ T TUC charges (Loss_cost + Cong_cost) | + = Due ISO |
| 509 | Hrly Ext. TSC MWHr | NUMBER(6,1) | MWh | Total Transmission Service Charge MWhs for External Transaction |  |
| 510 |  |  |  |  |  |
| 521 | Hrly Ext. TSC MWHr to Central Hudson | NUMBER(6,1) | MWh | Transmission Service Charge MWhs for External Transaction to Central Hudson |  |
| 522 | Hrly Ext. TSC MWHr to Con Ed | NUMBER(6,1) | MWh | Transmission Service Charge MWhs for External Transaction to Con Ed |  |
| 523 | Hrly Ext. TSC MWHr to LIPA | NUMBER(6,1) | MWh | Transmission Service Charge MWhs for External Transaction to LIPA |  |
| 524 | Hrly Ext. TSC MWHr to NYPA | NUMBER(6,1) | MWh | Transmission Service Charge MWhs for External Transaction to NYPA |  |
| 525 | Hrly Ext. TSC MWHr to O&R | NUMBER(6,1) | MWh | Transmission Service Charge MWhs for External Transaction to O&R |  |
| 526 | Hrly Ext. TSC MWHr to NYSEG | NUMBER(6,1) | MWh | Transmission Service Charge MWhs for External Transaction to NYSEG |  |
| 527 | Hrly Ext. TSC MWHr to NIMO | NUMBER(6,1) | MWh | Transmission Service Charge MWhs for External Transaction to NIMO |  |

End of row 1 (hour starting 0)  for Organization / LSE 1 transaction data, to be followed by hour starting 1-23, followed by next LSE for this organization, and continuing to LSE  (n)

IIIB.	Transmission Customers Data    - Transaction LBMP Energy Charges

| TRANSACTIONS LBMP ENERGY | Header for Transaction Information |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID | Name of the Organization |  |
| 400 | LSE name | VARCHAR2(50) | ID | Name of Load Serving Entity(for External Transaction this will be NULL) for the transaction sink |  |
| 500 | Trans_id | NUMBER | ID # | Unique transaction Identifier |  |
| 101 | Start day | DATE Format: MON/DD/YYYY | Date | Start Date |  |
| 102 | Start hour | NUMBER(2) | Hour | Start Hour |  |
| 511 | DAM LBMP Market MWHr | NUMBER(6,1) | MWh | Day ahead LBMP MWh amount | + = Energy Purchased |
| 512 | DAM LBMP Market Energy $ | NUMBER(15,2) | $$ | Day ahead energy component cost | + = Due ISO |
| 513 | DAM LBMP Market Loss $ | NUMBER(15,2) | $$ | Day ahead loss component cost | + = Due ISO |
| 514 | DAM LBMP Market Cong $ | NUMBER(15,2) | $$ | Day ahead cong component cost | + = Due ISO |
| 515 | DAM LBMP Market LBMP $ | NUMBER(15,2) | $$ | Total day ahead LBMP cost | + = Due ISO |
| 516 | R/T LBMP Market MWHr | NUMBER(6,1) | MWh | R/T LBMP MWh amount | + = Energy Purchased |
| 517 | R/T LBMP Market Energy $ | NUMBER(15,2) | $$ | R/T energy component cost | + = Due ISO |
| 518 | R/T LBMP Market Loss $ | NUMBER(15,2) | $$ | R/T loss component cost | + = Due ISO |
| 519 | R/T LBMP Market Cong $ | NUMBER(15,2) | $$ | R/T cong component cost | + = Due ISO |
| 520 | R/T LBMP Market LBMP $ | NUMBER(15,2) | $$ | Total R/T LBMP cost | + = Due ISO |
| 528 | DAM Bid Cost Guarantee | NUMBER(15,2) | $$ | Day ahead bid cost guarantee | + = Due Transaction Owner |
| 529 | R/T Bid Cost Guarantee | NUMBER(15,2) | $$ | Real time bid cost guarantee | + = Due Transaction Owner |

End of row 1 (hour starting 0)  for Organization / transaction1 data, to be followed by hour starting 1-23, followed by next Transaction for this organization, and continuing to Transaction(n)

IV.	Transmission Customers Data    - Ancillary Service Charges

| ANCILLARY SERVICES | Header for Ancillary Services Information |
| --- | --- |

| Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID | Name of the Organization |  |
| 400 | LSE name | VARCHAR2(50) | ID | Name of Load Serving Entity(for External Transaction this will be NULL) |  |
| 101 | Start day | DATE Format: MON/DD/YYYY | Date | Start Date |  |
| 102 | Start hour | NUMBER(2) | Hour | Start Hour |  |
| 600 | Hourly Ancillary Service Billing MWHr | NUMBER(6,1) | MWh |  |  |
| 601 | Hourly External ExportTransactions MWHr | NUMBER(6,1) | MWh |  |  |
| 602 | Hourly External Wheel Thru Transactions MWHr | NUMBER(6,1) | MWh |  |  |
| 603 | NTAC Rate for current month | NUMBER(15,2) | $$ | NYPA transmission access rate |  |
| 604 | NTAC Charge $ | NUMBER(15,2) | $$ | NTAC charge | + = Due ISO |
| 605 | Voltage Support Rate, $/MWh | NUMBER(15,2) | $$ | Voltage Support rate |  |
| 606 | Hrly VSS Charge$ | NUMBER(15,2) | $$ | hourly voltage support charge | + = Due ISO |
| 607 | S,SC&D MST Rate for current month | NUMBER(15,2) | $$ | ISO MST uplift rate |  |
| 608 | S,SC&D MST Charge $ | NUMBER(15,2) | $$ | ISO MST uplift charge | + = Due ISO |
| 609 |  |  |  |  |  |
| 610 | Hrly Reserve Chg$ | NUMBER(15,2) | $$ | Total hourly operating reserve charge | + = Due ISO |
| 611 | Residual Adjustment $ | NUMBER(15,2) | $$ | Hourly OATT Sch 1 residual adjustment | + = Due ISO |
| 612 | Hrly R&FR Charge $ | NUMBER(15,2) | $$ | Total hourly regulation charge | + = Due ISO |
| 613 | Black Start Charge $ | NUMBER(15,2) | $$ | Total hourly black start charge |  |
| 614 | S,SC&D OAT Rate for current month | NUMBER(15,2) | $$ | ISO OAT uplift rate |  |
| 615 | S,SC&D OAT Charge $ | NUMBER(15,2) | $$ | ISO OAT uplift charge | + = Due ISO |
| 616 |  |  |  |  |  |
| 617 | LRR Black Start Charge$ | NUMBER(15,2) | $$ | Daily total Local Reliability Black start charge | + = Due ISO |

End of row 1 (hour starting 0)  for Organization / LSE 1 ancillary data, to be followed by hour starting 1-23, followed by next LSE for this organization, and continuing to LSE  (n)

V. Transmission Congestion Contract  Holders Data

| TCC | Header for TCC Information |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID | Organization Name |  |
| 900 | TCC Contract ID | NUMBER | ID# | Transmission Congestion Contract ID |  |
| 101 | Start day | DATE Format: MON/DD/YYYY | Date | Start Date |  |
| 102 | Start hour | NUMBER(2) | Hour | Start Hour |  |
| 901 | Tcc credit | NUMBER(15,2) | $$ | Transmission Congestion Contract payment value | + = Due TCC holder |

End of row 1 (hour starting 0)   for Organization / TCC 1 data, to be followed by hour starting 1-23,  followed by next TCC for this organization and continuing to TCC  (n)

VI. Transmission Owner Data

| TRANSMISSION OWNERS - EXT TSC | Header for Transmission Owners Information |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 1000 | TP_name | VARCHAR2(50) | ID | Transmission Owner Name |  |
| 100 | TSC Org name | VARCHAR2(50) | ID | Transmission Customer of record for transaction imwhr |  |
| 400 | LSE name | VARCHAR2(50) | ID | Name of Load Serving Entity(for External Transaction this will be NULL) for the sink |  |
| 500 | Trans_id | NUMBER | ID # | Unique transaction Identifier |  |
| 101 | Start_day | DATE Format: MM/DD/YYYY | Date | Start Date |  |
| 102 | Start_hour | NUMBER(2) | Hour | Start Hour |  |
| 1002 | Ext_tsc_imwhr | NUMBER(6,1) | MWh | MWh of export transactions |  |

End of row 1(hour starting 0)  for Organization / TP 1 data, to be followed by hour starting 1-23, followed by next TP for this organization and continuing to TP(n)

VII. Transmission Owner Data – NYPA NTAC

| NYPA – NTAC | Header for Transmission Owners Information |
| --- | --- |

| ### Code | Parameter |  | Data Type | Element Type |  |  | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) |  |  | ID | Organization Name |  |  |
| 100 | TP_name |  | VARCHAR2(50) | ID |  |  | Transmission Owner Name |  |
| 101 | Start_day |  | DATE Format: MM/DD/YYYY | Date |  |  | Start Date |  |
| 102 | Start_hour |  | NUMBER(2) | Hour |  |  | Start Hour |  |
| 1003 | NTAC_Credit |  | NUMBER(15,2) | $$ |  |  | NTAC credit (applys only to NYPA) | + = Due TO |

VIII. Transmission Owner Data- Grandfathered Transaction Exempt from TSC

| TRANSMISSION OWNERS -Exempt TSC MWs | Header for Transmission Owners Information |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 1000 | TP_name | VARCHAR2(50) | ID | Transmission Owner Name |  |
| 500 | Trans_id | NUMBER | ID # | Unique transaction Identifier |  |
| 101 | Start_day | DATE Format: MM/DD/YYYY | Date | Start Date |  |
| 102 | Start_hour | NUMBER(2) | Hour | Start Hour |  |
| 1050 | Transaction User Ref | VARCHAR2(16) | ID |  |  |
| 1051 | PTID OF POI | NUMBER | ID | NYISO assigned point identifier of transaction point of injection |  |
| 1052 | PTID OF POW | NUMBER | ID | NYISO assigned point identifier of transaction point of withdrawel |  |
| 1053 | Exempt MWh | NUMBER(6,1) | MWh | MWh amount of transaction exempt from TSC |  |

End of row 1 (hour starting 0) for Transmission Provider / transaction1 data, to be followed by hour starting 1-23, followed by next Transaction for this Transmission Provider and continuing to Transaction (n).

VIII. Demand Reduction Programs

| Demand Reduction Programs | Header for Transmission Owners Information |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(80) | ID | Transmission Owner Name |  |
| 2000 | Demand Reduction Provider Name | VARCHAR2(80) | ID | Unique transaction Identifier |  |
| 2001 | Demand Reduction Provider PTID | NUMBER | ID # |  |  |
| 101 | Start_day | DATE Format: MON/DD/YY | Date | Start Date |  |
| 102 | Start_hour | NUMBER(2) | Hour | Start Hour |  |
| 2002 | DAM Demand Reduction Schedule MWHr | NUMBER(17,4) | ## | Reduction scheduled |  |
| 2003 | DAM Demand Reduction Actual MWHr | NUMBER(17,4) | ## | Reduction achieved |  |
| 203 | DAM LBMP (Generator) | NUMBER(15,2) | $$ | DAM LBMP price at pseudo-generator bus |  |
| 403 | DAM LBMP (Zonal) | NUMBER(15,2) | $$ | DAM Zonal LBMP price for LSE |  |
| 2004 | R/T LBMP (Generator – Time Weighted But Not Load Weighted) | NUMBER(15,2) | $$ | R/T LBMP price at pseudo-generator bus |  |
| 408 | R/T LBMP (Zonal) | NUMBER(15,2) | $$ | R/T Zonal LBMP price for LSE bus |  |
| 2005 | Demand Response Incentive $ | NUMBER(15,2) | $$ | Hourly program incentive payment |  |
| 2006 | Demand Response Reduction $ | NUMBER(15,2) | $$ | Hourly payment for reduction |  |
| 2007 | Demand Response Penalty $ | NUMBER(15,2) | $$ | Penalty charge for non-performance |  |
| 2008 | Demand Reduction Load Balancing $ | NUMBER(15,2) | $$ | Balancing charge for LSE load reduction |  |
| 2009 | Load Reduction Bid Guarantee $ | NUMBER(15,2) | $$ | Bid cost guarantee |  |

FILE TWO: Daily  Data:    DD-MON-YYYY

## Power Suppliers Data

| POWER SUPPLIERS DATE – AS OF: date | Generator Information header |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type |  | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID |  | Name of the Organization |  |
| 200 | Gen name | VARCHAR2(50) | ID |  | Name for the Generator |  |
| 201 | Gen Ptid | NUMBER(5) | ID |  | NYISO assigned Generator identifier |  |
| 101 | Start day | DATE Format: MM/YYYY | Date |  | Start Date |  |
| 300 | Day Ahead LBMP MWh | NUMBER(6,1) | MWh |  | Daily total day ahead LBMP MWh | + = Sale to the ISO |
| 301 | Day Ahead forward Energy $ |  |  |  | Daily total day ahead LBMP values | + = Due Generator |
| 302 | Day Ahead BPCG $ | NUMBER(15,2) | $$ |  | Daily dam minimum generation / start up payments | + = Due Generator |
| 303 | Integrated R/T Balancing MWh | NUMBER(6,1) | MWh |  | Daily total time weighted balancing MWhs | + = Sale to the ISO |
| 304 | Balancing Energy $ | NUMBER(15,2) | $$ |  | Daily balancing energy payment or charge | + = Due Generator |
| 305 | R/T BPCG $ | NUMBER(15,2) | $$ |  | Daily balancing minimum generation / start up payments | + = Due Generator |
| 306 | Voltage Support $ | NUMBER(15,2) |  | $$ | Daily Voltage payment based on in service time and fixed rate or fixed payment schedule for units under contract to supply installed capacity | + = Due Generator |
| 307 | VSS LOC $ | NUMBER(15,2) |  | $$ | Daily Lost opportunity cost value for units directed to hold a MW output to support voltage | + = Due Generator |
| 308 | Regulation Payment $ | NUMBER(15,2) |  | $$ | Daily Regulation Response Availability Payment | + = Due Generator |
| 309 | Regulation Charge $ | NUMBER(15,2) |  | $$ | Daily Regulation &Frequency Response Penalties | + = Due ISO |
| 310 | Operating Reserve Payment $ | NUMBER(15,2) |  | $$ | Daily Total Operating Reserve Service Settlement Payment | + = Due Generator |
| 311 | Black Start Daily Revenue Req’t | NUMBER(15,2) |  | $$ | Daily Black Start Revenue Requirement | + = Due Generator |
| 312 | Black Start Service Payment $ | NUMBER(15,2) |  | $$ | Black Start Service Payment | + = Due Generator |
| 313 | DAM Contract Balancing Payment $ | NUMBER(15,2) |  | $$ | Payment to make units whole for being dispatched below their day-ahead schedule out-of-merit | + = Due Generator |
| 314 | ELR DAM Contract Balancing Payment $ | NUMBER(15,2) |  | $$ | Payment to make units whole for being dispatched below their day-ahead schedule out-of-merit as ELR. | + = Due Generator |

End of row 1 of Generator data to be followed by next Generator for this organization, and continuing to Generator (n)

- Transmission Customers Data - LSE LBMP Energy and Transaction TUC  and TSC Charges

| LSE LBMP ENERGY AND TRANSACTIONS DAILY - AS OF: date | LSE Information header |
| --- | --- |

| ### Code | Parameter | Data Type |  | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) |  | ID | Name of the Organization |  |
| 400 | LSE name | VARCHAR2(50) |  | ID | Name of Load Serving Entity |  |
| 101 | Start day | DATE Format: MM/YYYY |  | Date | Start Date |  |
| 700 | Day Ahead Hrly LBMP MWh | NUMBER(6,1) |  | MWh | Daily LBMP MWs purchased Day-Ahead (Load bid MWs) | + = Purchase from the ISO |
| 701 | Fwd Energy $ | NUMBER(15,2) |  | $$ | Daily Energy component cost | + = Due ISO |
| 702 | Fwd Loss $ | NUMBER(15,2) |  | $$ | Daily Loss component cost | + = Due ISO |
| 703 | Fwd Cong $ | NUMBER(15,2) |  | $$ | Daily Congestion component cost | + = Due ISO |
| 704 | LSE Integrated R/T Balancing MWh by zone | NUMBER(6,1) |  | MWh | Daily Time weighted load estimate based on LSE forecast, adjusted for NYISO measured subzone load proportional with other LSE’s in this subzone, net from day ahead schedule and all bilateral transaction MWs | + = Purchase from the ISO |
| 705 | Balancing Energy $ | NUMBER(15,2) |  | $$ | Daily Energy component cost | + = Due ISO |
| 706 | Balancing Loss $ | NUMBER(15,2) |  | $$ | Daily Loss component cost | + = Due ISO |
| 707 | Balancing Cong $ | NUMBER(15,2) |  | $$ | Daily Congestion component cost | + = Due ISO |
| 750 | Day Ahead Scheduled Transactions | NUMBER(6,1) | MWh |  | Daily Total of all day ahead transactions MWh amount |  |
| 751 | Transaction Day Ahead Loss $ | NUMBER(15,2) | $$ |  | Daily Total of all transactions Loss component cost | + = Due ISO |
| 752 | Transaction Day Ahead Congestion $ | NUMBER(15,2) | $$ |  | Daily Total of all transactions Congestion component cost | + = Due ISO |
| 753 | Day Ahead TUC $ | NUMBER(15,2) | $$ |  | Daily Total transmission use charge (loss_cost + Cong_cost) | + = Due ISO |
| 754 | R/T Scheduled Transactions | NUMBER(6,1) | MWh |  | Daily Total of all R/T transaction MWhs |  |
| 755 | R/T Balancing Loss $ | NUMBER(15,2) | $$ |  | Daily Total cost of losses on transactions | + = Due ISO |
| 756 | R/T Balancing Congestion $ | NUMBER(15,2) | $$ |  | Daily Total cost of congestions on transactions | + = Due ISO |
| 757 | Hrly R/T TUC $ | NUMBER(15,2) | $$ |  | Daily Total TUC charges (Loss_cost + Cong_cost) | + = Due ISO |

End of row 1 for Organization / LSE 1 data, to followed by next LSE for this organization and continuing to LSE  (n)

IIB. Transmission Customers Data - Transaction LBMP Energy

| TRANSACTIONS LBMP ENERGY DAILY - AS OF: date | Information header |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID | Name of the Organization |  |
| 400 | LSE name | VARCHAR2(50) | ID | Name of Load Serving Entity(for External Transaction this will be NULL) for the transaction sink |  |
| 101 | Start Day | DATE Format: MM/YYYY | Date | Start Date |  |
| 758 | DAM LBMP Market MWHr | NUMBER(6,1) | MWh | Day ahead LBMP MWh amount | + = Purchase from the ISO |
| 759 | DAM LBMP Market Energy $ | NUMBER(15,2) | $$ | Day ahead energy component cost | + = Due ISO |
| 760 | DAM LBMP Market Loss $ | NUMBER(15,2) | $$ | Day ahead loss component cost | + = Due ISO |
| 761 | DAM LBMP Market Cong $ | NUMBER(15,2) | $$ | Day ahead cong component cost | + = Due ISO |
| 762 | DAM LBMP Market LBMP $ | NUMBER(15,2) | $$ | Total day ahead LBMP cost | + = Due ISO |
| 763 | R/T LBMP Market MWHr | NUMBER(6,1) | MWh | R/T LBMP MWh amount | + = Purchase from the ISO |
| 764 | R/T LBMP Market Energy $ | NUMBER(15,2) | $$ | R/T energy component cost | + = Due ISO |
| 765 | R/T LBMP Market Loss $ | NUMBER(15,2) | $$ | R/T loss component cost | + = Due ISO |
| 766 | R/T LBMP Market Cong $ | NUMBER(15,2) | $$ | R/T cong component cost | + = Due ISO |
| 767 | R/T LBMP Market LBMP $ | NUMBER(15,2) | $$ | Total R/T LBMP cost | + = Due ISO |
| 768 | DAM Bid Cost Guarantee | NUMBER(15,2) | $$ | Day ahead bid cost guarantee | + = Due Transaction Owner |
| 769 | R/T Bid Cost Guarantee | NUMBER(15,2) | $$ | Real time bid cost guarantee | + = Due Transaction Owner |

End of row 1 for Organization / LSE 1 data, to followed by next LSE for this organization and continuing to LSE  (n)

III.	Transmission Customers Data    - Ancillary Service Charges

| ANCILLARY SERVICES DAILY- AS OF: date | Ancillary Services Information header |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID | Name of the Organization |  |
| 400 | LSE name | VARCHAR2(50) | ID | Name of Load Serving Entity(for External Transaction this will be NULL) |  |
| 101 | Start day | DATE Format: MM/YYYY | Date | Start Date |  |
| 800 | Ancillary Service Billing MWHr | NUMBER(6,1) | MWh | Daily billing mwhr |  |
| 801 | External ExportTransactions MWHr | NUMBER(6,1) | MWh | Daily External ExportTransactions MWHr |  |
| 802 | Hourly External Wheel Thru Transactions MWHr | NUMBER(6,1) | MWh | Daily Hourly External Wheel Thru Transactions MWHr |  |
| 803 | NTAC Charge $ | NUMBER(15,2) | $$ | Daily NTAC charge | + = Due ISO |
| 804 | VSS Charge$ | NUMBER(15,2) | $$ | Daily voltage support charge | + = Due ISO |
| 805 | S,SC&D MST Charge $ | NUMBER(15,2) | $$ | Daily ISO MST uplift charge | + = Due ISO |
| 806 | Reserve Chg$ | NUMBER(15,2) | $$ | Daily Total operating reserve charge | + = Due ISO |
| 807 | R&FR Charge $ | NUMBER(15,2) | $$ | Daily Total regulation charge | + = Due ISO |
| 808 | Black Start Charge $ | NUMBER(15,2) | $$ | Daily Total black start charge | + = Due ISO |
| 809 | S,SC&D OAT Charge $ | NUMBER(15,2) | $$ | Daily ISO OAT uplift charge | + = Due ISO |
| 810 | LRR Mingen Charge$ | NUMBER(15,2) | $$ | Daily Local Reliability Uplift charge | + = Due ISO |
| 811 | LRR Black Start Charge $ | NUMBER(15,2) | $$ | Daily Local Reliability Black Start Charge | + = Due ISO |
| 812 | NYISO-wide Uplift Charge $ | NUMBER(15,2) | $$ | Daily NYISO-wide Uplift charge from BPCG | + = Due ISO |
| 813 | Residual Adjustment $ | NUMBER(15,2) | $$ | Daily NYISO-wide Uplift charge from settlement residuals | + = Due ISO |
| 814 | Demand Program Uplift $ | NUMBER(15,2) | $$ | DAM Price Responsive Load Program Uplift charge | + = Due ISO |

End of row 1 for Organization / LSE 1 ancillary data, to be followed by next LSE for this organization and continuing to LSE  (n)

IV. Transmission Congestion Contract  Holders Data

| TCC DAILY – AS OF: date | TCC Information header |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(50) | ID | Organization Name |  |
| 900 | TCC Contract ID | NUMBER | #ID | Transmission Congestion Contract ID |  |
| 101 | Start day | DATE Format: MM/YYYY | Date | Start Date |  |
| 903 | Tcc credit | NUMBER(15,2) | $$ | Daily Transmission Congestion Contract payment value | + = Due TCC Holder |

End of row 1 for Organization / TCC 1 data, to be  followed by next TCC for this organization and continuing to TCC  (n)

V. Demand Reduction Programs

| Demand Reduction Programs | Header for Transmission Owners Information |
| --- | --- |

| ### Code | Parameter | Data Type | Element Type | Description | Payment Made To/From |
| --- | --- | --- | --- | --- | --- |
| 100 | Org name | VARCHAR2(80) | ID | Transmission Owner Name |  |
| 2000 | Demand Reduction Provider Name | VARCHAR2(80) | ID | Unique transaction Identifier |  |
| 2001 | Demand Reduction Provider PTID | NUMBER | ID # |  |  |
| 101 | Start_day | DATE Format: MON/DD/YY | Date | Start Date |  |
| 2010 | Demand Reduction MWHr | VARCHAR4(6) | ID | Reduction settled |  |
| 2011 | Demand Response Incentive $ | NUMBER(15,2) | $$ | Daily program incentive payment |  |
| 2012 | Demand Response Reduction $ | NUMBER(15,2) | $$ | Daily payment for reduction |  |
| 2013 | Demand Response Penalty $ | NUMBER(15,2) | $$ | Penalty charge for non-performance |  |
| 2014 | Demand Reduction Load Balancing $ | NUMBER(15,2) | $$ | Balancing charge for LSE load reduction |  |
| 2015 | Load Reduction Bid Guarantee $ | NUMBER(15,2) | $$ | Bid cost guarantee |  |

FILE Three: Pre-Bill  Data:    Feed Number: ##

| ### Code | Parameter | Data Type | Element Type | Description |
| --- | --- | --- | --- | --- |
| 5010 | ORG_NAME | VARCHAR(50) | ID | Name of the Organization |
| 5020 | DEVICE_DESC | VARCHAR(50) | ID | Name for the device (load bus, tie line, loss component or generator) |
| 5030 | DEVICE_PTID | NUMBER(6) | ID | NYISO assigned device identifier |
| 5040 | DATE | DATE Format: DD-MMM-YY | Date | Date in UT |
| 5050 | HOUR_BEG | INTEGER | Hour | Hour Beginning in UT |
| 5060 | MWH | NUMBER(20,4) | MWh | Hourly reading |
| 5070 | SOURCE_ID | NUMBER(6) | ID | NYISO assigned identifier indicating source of data. |
| 5080 | SOURCE_DESC | VARCHAR2(50) | ID | Text description of source of data |
| 5090 | QUALITY | INTEGER | ## | Data quality indicator: 1 = Forecasted (for Load Data) / Estimate (for Tie Line or Gen Data) 2 = Integrated Instantaneous Analog 3 = Integrated Instantaneous Digital 4 = State Estimated 5 = RTU Data 6 = Unadjusted Billing Meter 7 = Manual Entry 8 = Calculated 9 = Adjusted Billing Meter 0 = Cannot be determined |

The coma separated format will look like this:

File One:  HOURLY DATA

Line 1:		POWER SUPPLIERS

Line 2:		Codes

Line 3:		Text string of charges

Line 4:		raw data for power suppliers

Line X:		raw data for power suppliers

LineX+1:	LSE LBMP ENERGY

LineX+2:	Codes

LineX+3:	Text  string of charges

LineX+4:	raw data for lse lbmp energy

LineX:		raw data for lse lbmp energy

LineX+1:	TRANSACTIONS

LineX+2:	Codes

LineX+3	Text  string of charges

LineX+4:	raw data for transactions

LineX:		raw data fro transaction

LineX+1:	TRANSACTIONS LBMP ENERGY

LineX+2:	Codes

LineX+3	Text  string of charges

LineX+4:	raw data for transactions

LineX:		raw data fro transaction

LineX+1:	ANCILLARY SERVICES

LineX+2:	Codes

LineX+3:	Text string of charges

LineX+4:	raw data for ancillary charges

LineX:		raw data for ancillary charges

LineX+1:	TCC

LineX+2:	Codes

LineX+3:	Text string of charges

LineX+4:	raw data for TCC

LineX:		raw data for TCC

LineX+1:	TRANSMISSION OWNERS – EXT TSC

LineX+2:	Codes

LineX+3:	Text string of charges

LineX+4:	raw data for transmission owners

LineX:		raw data for transmission owners

LineX+1:	NYPA - NTAC

LineX+2:	Codes

LineX+3:	Text string of charges

LineX+4:	raw data for transmission owners

LineX:		raw data for transmission owners

LineX+1:	TRANSMISSION OWNERS – EXEMPT GRANDFATHERED TSC

LineX+2:	Codes

LineX+3:	Text string of charges

LineX+4:	raw data for transmission owners

LineX:		raw data for transmission owners

File Two:  Power Supplier SCD interval base point mw

Line 1:		POWER SUPPLIERS UNIT LOADING

Line 2:		Codes

Line 3:		Text string of codes

Line 4:		raw data for power suppliers

Line X:		raw data for power suppliers

File Three:  Daily DATA

Line 1:		POWER SUPPLIERS DAILY - AS OF: date

Line 2:		Codes

Line 3:		Text string of charges

Line 4:		raw data for power suppliers

Line X:		raw data for power suppliers

LineX+1:	LSE LBMP ENERGY AND TRANSACTIONS DAILY - AS OF: date

LineX+2:	Codes

LineX+3:	Text  string of charges

LineX+4:	raw data for lse lbmp energy and transactions

LineX:		raw data for lse lbmp energy and transactions

LineX+1:	TRANSACTIONS LBMP ENERGY DAILY - AS OF: date

LineX+2:	Codes

LineX+3:	Text  string of charges

LineX+4:	raw data for lse lbmp energy and transactions

LineX:		raw data for lse lbmp energy and transactions

LineX+1:	ANCILLARY SERVICES DAILY - AS OF: date

LineX+2:	Codes

LineX+3:	Text string of charges

LineX+4:	raw data for ancillary charges

LineX:		raw data for ancillary charges

LineX+1:	TCC DAILY - AS OF: date

LineX+2:	Codes

LineX+3:	Text string of charges

LineX+4:	raw data for TCC

LineX:		raw data for TCC

LineX+1:	TRANSMISSION OWNERS DAILY - NTAC and Cong Credit - AS OF: date

LineX+2:	Codes

LineX+3:	Text string of charges

LineX+4:	raw data for transmission owners

LineX:		raw data for transmission owners