FIRM PIPELINE CAPACITY CONNECTED WITH SOUTHERN CALIFORNIA GAS COMPANY

| Delivery Point | Delivering Pipeline and Capacity (MMcfd) | SoCalGas Take-Away Capacity (MMcfd) |
| --- | --- | --- |
| Cole Levee (Gosford) | Occidental 200 | Incl. in WR |
| Kern River Station | PG&E Expansion 600 | Incl. in WR |
| Wheeler Ridge | Kern River 700 | 680 (1) (2) (3) |
|  | Mojave Incl. in Topock(4) |  |
| Hector Road | Mojave Incl. in Topock(4) | 50 |
| North Needles | Transwestern 1,090 | 750 |
| South Needles (Topock) | El Paso 2,230 | 540 |
|  | Transwestern Incl. in No. Needles(5) |  |
| Blythe (Ehrenberg) | El Paso 1,210 | 1,210 |
| Line 85 | Production 150 | 150 |
| North Coastal | Production 120 | 120 |
| TOTAL | 6,300 | 3,500 |

(1)  Take-away from PG&E at Kern River Station is 600 but is limited to 520 MMcfd by Line 225 capacity from KRS to Wheeler Ridge with 650 psig of pressure from PG&E.

(2)  Kern/Mojave meter currently set to 500 MMcfd.

(3)  Total SoCalGas take-away capacity of 680 MMcfd from PG&E, Occidental and Kern/Mojave interconnects assumes minimum flow of 160 MMcfd from Kern/Mojave interconnect.

(4)  Mojave's 400 MMcfd capacity is connected with SoCalGas at both Hector Road and Wheeler Ridge.  Insofar as Mojave is supplied out of El Paso's 2,230 MMcfd at Topock, Mojave capacity is not included as additional delivering pipeline capacity to avoid double-counting.

(5)  340 MMcfd of Transwestern's 1,090 MMcfd is connected with SoCalGas at both North Needles or Topock.