# Additional Definitions

AFTS – Advanced Fiber Transport Solutions
DW – Dense Waves

POP – Point of Presence

# Section 8

8.1 General

Enron Broadband Services is a leading provider of high quality, broadband Internet content and applications.  The company’s business model combines the power of the Enron Intelligent Network, Enron’s Broadband Operating System, bandwidth trading and intermediation services, and high-bandwidth applications, to fundamentally improve the experience and functionality of the Internet.

8.2 Advanced Fiber Transport Solutions (AFTS) Services

8.2.1 Description

Advanced Fiber Transport Solutions (AFTS) are dense waves (DW) of capacity between two Enron Points of Presence (POP).  AFTS is available in DW-12 and DW-48 capacity increments.  DW-12 is a dense wave of 622 megabit per second (Mbps) transmission capacity and DW-48 is a dense wave of 2.5 gigabit per second (Gbps) transmission capacity.  The dense waves transport ATM, IP, and SONET traffic seamlessly in the same channel.

8.2.3 Connecting Points

Enron will terminate the service at an Enron specified POP.  It is the customer’s responsibility to connect to the demarcation point in the Enron POP.

8.2.4 Performance Specifications

The AFTS service shall be designed and maintained per manufacturer’s recommendations to operate with nominal error-free performance.  The DW-12 and DW-48 service shall be deemed acceptable when each city pair section performance bit error ratio meets or exceed manufacturer’s specifications of 1x10-15, without the occurrence of any protection switching or frame loss events.

8.2.5 Performance and Availability

DW-12 and DW-48 service will meet or exceed the following error performance and availability objectives between Customer-provided network interface points, exclusive of Enron termination equipment.

Error Performance			Availability

Mileage			(% Error Free Seconds)		(% Availability)

<250				99.95%				99.99%

250-1000			99.91%				99.98%

>1000				99.86%				99.97%

Error performance is defined in percent of error free seconds which is the percentage of total seconds that when measured over a consecutive 24-hour period do not contain bit errors.

Availability is defined as the percent of total time the service is available when measured over a 365-day period.  Service is considered unavailable when the bit error ratio, measured at the DS-3 or STS-1 rate, is worse that 1 x 10-3 for ten consecutive seconds.  Unavailability is inclusive of this time and continues until service restoration which begins after ten consecutive seconds having a bit error ratio better than 1 x 10-6.  Service shall not be considered unavailable when lack of availability is due to Customer’s action or inaction.  The availability objective applies to normal operation and routine maintenance of the service and excludes long-term outage due to failure of a catastrophic nature, including incidental damage to or severance or outside fiber optic cable plant.

8.2.6 Price

AFTS is available on a 24-hour per day, 7 days per week basis between two Enron Points of Presence (POP).  Monthly Recurring Rates for this service are based on term and volume commitment.  The following table outlines the above-mentioned pricing matrix.

DW-12 Price: Route miles * rate per DS-0 Mile * 8,640 DS-0s in DW-12

DW-48 Price: Route miles * rate per DS-0 Mile * 32,256 DS-0s in DW-48

|  | Price per DS-0 mile |  |
| --- | --- | --- |
| Term | DW-12 Service | DW-48 Service |
|  | One DW-12 Channel | One DW-48 Channel |
| 1 year | 0.032 | 0.020 |
| 3 year | 0.029 | 0.018 |
| 5 year | 0.026 | 0.016 |
| 10 year | 0.021 | 0.013 |
|  | ## Two DW-12 Channels | Two DW-48 Channels |
| 1 year | 0.029 | 0.018 |
| 3 year | 0.026 | 0.016 |
| 5 year | 0.022 | 0.014 |
| 10 year | 0.018 | 0.011 |
|  | Three DW-12 Channels | Three DW-48 Channels |
| 1 year | 0.026 | 0.016 |
| 3 year | 0.022 | 0.014 |
| 5 year | 0.019 | 0.012 |
| 10 year | 0.014 | 0.009 |
|  | Four DW-12 Channels | Four DW-48 Channels |
| 1 year | 0.022 | 0.014 |
| 3 year | 0.019 | 0.012 |
| 5 year | 0.016 | 0.010 |
| 10 year | 0.011 | 0.007 |
|  | Five DW-12 Channels | Five DW-48 Channels |
| 1 year | 0.019 | 0.012 |
| 3 year | 0.016 | 0.010 |
| 5 year | 0.013 | 0.008 |
| 10 year | 0.008 | 0.005 |
|  | Six DW-12 Channels | Six DW-48 Channels |
| 1 year | 0.016 | 0.010 |
| 3 year | 0.013 | 0.008 |
| 5 year | 0.010 | 0.006 |
| 10 year | 0.005 | 0.003 |
|  | Seven DW-12 Channels | Seven DW-48 Channels |
| 1 year | 0.013 | 0.008 |
| 3 year | 0.010 | 0.006 |
| 5 year | 0.006 | 0.004 |
| 10 year | 0.004 | 0.0025 |

Non Recurring Rates:  $15,000 per origination and per termination sites

$5,000 per pass through termination or regeneration sites