======================================================================

Control Number: 9800110

Category:  NSPS

Region:    Region 3

Date:      07/02/1998

Title:     Alternative Monitoring Approval

Recipient: Faggert, P.F.

Author:    McGuigan, David

Comments:

Abstract:

Q: Will EPA approve a custom fuel monitoring schedule,

waive fuel nitrogen monitoring requirements, and approve

the use of an alternative location for fuel sampling?

A: Yes, EPA will approve these alternatives under certain

specified conditions.

Letter:

In Reply Refer To: 3AP13

CERTIFIED MAIL

RETURN RECEIPT REQUESTED

P.F. Faggert

Team Leader

Environmental Regulations and Permits

Virginia Power

Innsbrook Technical Center

5000 Dominion Boulevard

Glen Allen, Virginia 23060

Re: Various Alternative Requests under NSPS Subpart GG

Dear Ms. Faggert:

The Philadelphia Regional Office of the U.S. Environmental

Protection Agency (Region III) has received and reviewed

your requests, dated April 29, 1998, for alternative

methodologies/procedures, as provided under the New Source

Performance Standards (NSPS) program, to be applied to

stationary gas turbines subject to Subpart GG requirements

at your Darbytown Station.  You had requested alternatives

to three separate Subpart GG conditions, namely: an

alternative, custom fuel monitoring schedule for

pipeline-quality natural gas fuel; a waiver of the

nitrogen-in-fuel  monitoring requirement; and, finally, an

allowance for obtaining fuel samples for analysis at a

different location in the gas pipeline in order to minimize

the sampling/analysis necessary for compliance with Subpart

GG at a number of gas turbine stations. In regard to the

first two requests, EPA does have a National Policy, as you

indicated, which allows affected facilities that combust

pipeline-quality natural gas fuel to monitor the fuel

sulfur content at a reduced rate, as specified in the

Policy and upon approval by EPA, and to be granted a waiver

from the fuel nitrogen monitoring requirement as natural

gas fuel has been determined to contain no fuel bound

nitrogen and having free nitrogen which does not contribute

appreciably to the formation of Nox emissions.

As your Darbytown Station turbines combust pipeline quality

natural gas fuel and the fuel quality has been established,

based on EPA's current understanding of the situation, as

equivalent to that utilized by the Doswell Limited

Partnership turbines which are already on a custom fuel

monitoring schedule, EPA, hereby, approves the use of the

fuel sulfur monitoring schedule contained in EPA's August,

1987 Policy for the Darbytown Station turbines and waives

the fuel nitrogen monitoring requirement for your natural

gas fuel as allowed under the same Policy.

EPA is also, hereby, approving the use of natural gas

samples taken at your Chesterfield Power Station, for

analytical purposes, to fulfill the fuel analysis

requirement for the Darbytown turbines as the two stations

both combust identical fuel from the same gas pipeline.  As

you have indicated, EPA had approved a similar sampling

allowance for Transcontinental Gas Pipeline Corporation

under specified conditions which will now apply to Virginia

Power as well, namely, that your Company will supply one

month of data, prior to the initiation of the reduced

monitoring schedule, from daily fuel monitoring to verify

the sulfur content of the fuel and that Virginia Power will

follow the schedule specified in EPA's August 14, 1987 memo

regarding custom fuel monitoring schedules under NSPS

Subpart GG.  It is EPA's understanding, based on our review

of the pipeline schematic supplied by Virginia Power, that

Doswell, Darbytown and Chesterfield Stations utilize the

same fuel supply for turbine operation and that no other

piping is introduced into the pipeline system at or between

these described Stations which would have the effect of

mixing any other fuel or material into the natural gas fuel

being combusted in the turbines so as to alter the fuel

analysis between the mentioned Stations.

If you should have any questions or comments in regard to

these approvals, do not hesitate to contact me, at (215)

814-2158, or James W. Hagedorn, of my staff, at (215)

814-2161.

Sincerely,

David B. McGuigan, Ph.D., Chief

CAA Area II Enforcement Branch

cc: Charles King, VaDEQ-Piedmont Regional Office

Jordan Spooner, OECA

======================================================================

Control Number: 9800109

Category:  NSPS

Region:    Region 3

Date:      05/27/1998

Title:     Waiver of Nitrogen Monitoring

Recipient: Hoffman, P.M.

Author:    McGuigan, David

Comments:

Abstract:

Q: Will EPA approve a waiver of the fuel nitrogen

monitoring requirement under NSPS GG?

A: Yes, under certain circumstances a waiver can be

granted under current EPA National Policy.

Letter:

In Reply Refer To: 3AP13

CERTIFIED MAIL

RETURN RECEIPT REQUESTED

Mr. P.M. Hoffman

Environmental Coordinator

Columbia Gas Transmission

1700 MacCorkle Ave. SE

Charleston, West Virginia 25314

Re: Approval of Nitrogen Content Monitoring Waiver Under

NSPS Subpart GG

Dear Mr. Hoffman:

The Philadelphia Regional Office of the U.S. Environmental

Protection Agency (EPA), Region III, has received and

reviewed your letter, dated April 30, 1998, requesting a

waiver of the NSPS Subpart GG requirement for fuel nitrogen

monitoring under Section 60.334(b) at the stationary gas

turbines in the Goochland Compressor Station.  Based on the

statements made in your referenced letter, EPA, hereby,

grants your requested waiver as the Goochland gas turbines

only combust, and are only allowed to combust, pipeline

quality natural gas fuel.  By Policy, dated August 14,

1987, EPA has already determined that nitrogen monitoring

can be waived for pipeline-quality natural gas since there

is no fuel bound nitrogen and the free nitrogen does not

contribute appreciably to Nox emissions.

If you should have any comments or questions in regard to

this matter, do not hesitate to contact me at (215)

566-2158 or James W. Hagedorn, of my staff, at (215)

566-2161.

Sincerely,

David B. McGuigan, Ph.D., Chief CAA Area II Enforcement

cc: Cheryl Mayo, VaDEQ-Piedmont Regional Office

======================================================================