ENERGY REPORTING NETWORK

KBR TOWER  SUITE 3690

601 JEFFERSON ST

HOUSTON, TX 77002

TEL:	713 . 651. 8006				FAX: 281 - 754 4934				  EMail:gasfax@usa.net

GASFAX®				                   WELLFAXsm					POWERFAXsm

REPORT NUMBER 504

DATE: July 9, 2001

This is Report Number 504 for inventories of Thursday, July 7, 2001.

Table A

| Imputed U.S. Natural Gas Demand |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Current Week BCF |  | Prior Week BCF |  | Season Avg, BCF |  |
|  | BCFW | BCFD | BCFW | BCFD | BCFW | BCFD |
| Net Imports PL Gas | 73.9 | 10.5 | 72.6 | 10.4 | 71.6 | 10.2 |
| LNG Imports | 0 | 0 | 5.2 | 0.7 | NA | NA |
| Net GG Storage Chge | 84.1 | 12.0 | 85.7 | 12.2 | 72.4 | 10.3 |
| Dry Marketed Gas | 357.0 | 51.0 | 357.0 | 51.0 | 352.7 | 50.4 |
| Apparent Demand | 346.8 | 49.5 | 349.1 | 49.9 | 351.9 | 50.3 |

# Please note we have added LNG imports to the Contiguous U.S. natural gas supply to calculate Apparent Demand. Canadian natural gas re-imports are already accounted for in the data, so need no adjustment. Mexican imports (U.S. exports) were 3% of Gross Pipeline gas imports and 2.9% of all imports, including LNG in 2000. Mexican imports (U.S. exports) are within the estimated accuracy of U.S. gas production estimates, so will be ignored in the calculations of Apparent Demand.

# The season average has not been recalculated to reflect the addition of LNG supplies, but will be.  Apparent Demand, in the new calculations, is down 0.7% (2.3 BCFW) from last week; net imports were up 1.8%, while GG storage injections were off 1.9% for the week.

# Executive Summary

- LNG imports have been added to the U.S. total to make the Apparent demand calculation.

2.	GG injections remained fairly high; a comparison has been put in the graphics section so you can. see how EIA and AGA June injections look over the past 11 years.

Table B

LNG Deliveries

| U.S. LNG Deliveries, Gross Gas |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Region | Storage Capacity | Send out Capability | Week Receipts | Month Cumulative | Cumulative Receipts | Cumulative Send Out |
|  | MMCF | MMCFD | MMCF | MMCF | MMCF | MMCF |
| 1 NE | 3,500 Active; 5,000 Start 2002 | 800 Active; 1,800 in 2002 | 0 | 0 | 56,700 | 55,500 |
| 4 SE | 4,200 Start 2002 | NA | NA | NA | NA | NA |
| 5 SW | 6,300 Active | 700 | 0 | 0 | 89,750 | 88,000 |
| Total U.S. | 9,800 Active | 1,500 | 0 | 0 | 146,450 | 143,500 |

This week neither Cabot nor CMS received any cargoes, apparently because of the July 4th holiday that would have incurred holiday pay scales to receive and unload. This week’s special topic addresses LNG and Pipeline natural gas imports and exports from 1995-2000, according to EIA data. The purpose is to quantify natural gas net imports for all gas forms and delivery modes.

Summary and Rankings

Sections B & C:  Weekly Regional Gross Gas Inventory Change and  Comparison to EOS (End Of Season) GasFax®  3 Year Average Inventory

Table C

| Weekly Ranking | Weekly Change BCFW |  | Avg Daily Change BCFDxc |  | Avg Change To Meet EOS Levels BCFW |  | Avg Change To Meet EOS Levels BCFD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Current | Prior | Current | Prior | ## Current | Prior | Current | Prior |
| North America | 89.9 | 91.1 | 12.8 | 13.0 | 71.4 | 72.1 | 10.2 | 10.3 |
| United States | 84.1 | 85.7 | 12.0 | 12.2 | 67.2 | 67.9 | 9.6 | 9.7 |
| Region 2 ENC | 27.3 | 34.4 | 3.9 | 4.9 | 25.2 | 25.2 | 3.6 | 3.6 |
| Region 1 NE | 26.5 | 32.6 | 3.8 | 4.7 | 14.7 | 15.4 | 2.1 | 2.2 |
| Region 5 SW | 16.0 | 11.2 | 2.3 | 1.6 | 14.7 | 14.7 | 2.1 | 2.1 |
| Region 7 PAC | 7.7 | 5.0 | 1.1 | 0.7 | 4.9 | 4.9 | 0.7 | 0.7 |
| Region 3 WNC | 5.1 | 3.5 | 0.7 | 0.5 | 6.3 | 5.6 | 0.9 | 0.9 |
| Region 6 MTN | 3.8 | 2.2 | 0.5 | 0.3 | 3.5 | 3.5 | 0.5 | 0.5 |
| Region 4 SE | 3.4 | 2.1 | 0.5 | 0.3 | 2.8 | 2.8 | 0.4 | 0.4 |

With exception of Region 3 (WNC), all regions injected either “target” needs or substantially more. Region 2 (ENC) was right at its average daily rates, but this is more a result of reservoir geology and large pipeline supplies via Alliance than any other reason. Region 1 (NE) put about 60% more in than target needs.

Table D

Section D:	Weekly Regional Working Gas Inventories and EOS Comparison

| GasFax® Regional Weekly Working Gas Change and End Of Season Inventory Comparison |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Current and Prior Week Inventories BCF |  | End Of Season (EOS) Inventory Comparison |  | Average Change Needed To Meet EOS Inventory |  |  |  |
| Weekly Ranking | Current Week | Prior Week | EOS Inventory BCF | Current Inventory % of EOS | Current BCFW | Prior BCFW | Current BCFD | Prior BCFD |
| North America | 2579 | 2489 | 3762 | 69 | 69.3 | 70.7 | 9.9 | 10.1 |
| United States | 2349 | 2265 | 3458 | 68 | 65.1 | 66.5 | 9.3 | 9.5 |
| ## Region 2 ENC | 1041 | 1014 | 1480 | 70 | 25.9 | 25.9 | 3.7 | 3.7 |
| Region 1 NE | 394 | 367 | 639 | 62 | 14.7 | 15.4 | 2.1 | 2.2 |
| Region 5 SW | 471 | 454 | 689 | 68 | 12.6 | 13.3 | 1.8 | 1.9 |
| ## Region 3 WNC | 128 | 123 | 390 | 55 | 6.3 | 5.6 | 0.9 | 0.9 |
| Region 7 PAC | 316 | 308 | 234 | 81 | 4.2 | 6.3 | 0.6 | 0.7 |
| Region 6 MTN | 111 | 107 | 168 | 66 | 3.5 | 3.5 | 0.5 | 0.5 |
| ## Region 4 SE | 119 | 115 | 162 | 73 | 2.8 | 2.8 | 0.4 | 0.4 |

Current week WG “needs” are somewhat different than GG because of the Base Gas that is included in the GG numbers. Theoretically they should be the same, but for lots of reasons they seem to differ, including reclassifications (formal or just temporary), operator guesstimates and so forth. This is why we don’t put a lot of emphasis on WG.

Table E

# Canadian Alberta Border and Total Pipeline Exports

| Alberta Weekly Border Exports | Current Volume |  |  | Prior Volume |  |  | Current Per Cent |  | Prior Per Cent |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BCFW | BCFD | BCFW |  | BCFD | Deliveries |  | Change | Deliveries | Change |
| Empress | 38.7 | 9.7 | 38.6 |  | 5.5 | 45.8 |  | 0.3 | 46.1 | 4.0 |
| McNeill | 15.1 | 2.2 | 14.9 |  | 2.1 | 17.9 |  | 1.3 | 17.8 | (0.7) |
| ABC | 14.8 | 2.1 | 14.9 |  | 2.1 | 17.5 |  | (0.7) | 17.8 | (3.3) |
| Net Other Borders | 0.2 | 0.02 | 0.1 |  | 0.01 | 0.2 |  | 100.0 | 0.1 | 0.0 |
| Intra Prov + Storage | 15.5 | 2.2 | 15.1 |  | 2.2 | 18.3 |  | 2.7 | 18.0 | 4.1 |
| Net Storage Change | 7.9 | 1.1 | 7.3 |  | 1.0 | 9.3 |  | 8.2 | 8.7 | 4.3 |
| Total Nova Delivery | 84.5 | 12.1 | 83.7 |  | 12.0 | 100.0 |  | 9.6 | 100.0 | 1.6 |
| Net Alberta Deliveries | 69.0 | 9.9 | 68.6 |  | 9.8 | 81.7 |  | 0.6 | 82.0 | 1.0 |
| Canadian Weekly Exports to U.S. | Current Volume |  | Prior Volume |  |  | Current Per Cent |  |  | Prior Per Cent |  |
|  | BCFW | BCFD | BCFW |  | BCFD | Exports |  | Change | Exports | Change |
| Kingsgate BC (ABC) | 14.8 | 2.1 | 14.9 |  | 2.1 | 20.0 |  | (0.7) | 20.5 | (3.3) |
| Sumas (Huntingdon) | 6.4 | 0.9 | 5.4 |  | 0.8 | 8.7 |  | 18.5 | 7.4 | 20.4 |
| Monchy SK (McNeill) | 15.1 | 2.2 | 14.9 |  | 2.1 | 20.4 |  | 1.3 | 20.5 | (0.07) |
| Emerson 1 & 2 (Viking + GLTS) | 16.6 | 2.4 | 16.3 |  | 2.3 | 22.5 |  | 1.8 | 22.5 | 5.2 |
| Niagra ON | 5.2 | 0.7 | 5.4 |  | 0.8 | 7.0 |  | (3.7) | 7.4 | 14.9 |
| Iroquois ON | 5.8 | 0.8 | 5.9 |  | 0.8 | 7.9 |  | (1.7) | 8.1 | 7.3 |
| Other TCPL Ports: (Chippawa, etc) | 1.5 | 0.2 | 2.0 |  | 0.3 | 2.0 |  | (25.0) | 2.5 | 11.1 |
| Sable Island (M&NE) | 3.4 | 0.5 | 2.8 |  | 0.4 | 4.6 |  | 21.4 | 3.9 | (17.6) |
| Alliance PL Net to US | 5.1 | 0.7 | 5.0 |  | 0.7 | 6.9 |  | 2.0 | 6.9 | (2.0) |
| Total Exports to U.S. | 73.9 | 10.5 | 72.6 |  | 10.3 | 100.0 |  | 1.8 | 100.0 | 1.8 |

* Source: National Energy Board, Canada, TransCanada Pipelines, West Coast Energy

## Alberta border deliveries had little change for the week (six tenths of a per cent gain), but net imports by the U.S. , including Alberta direct exports, were up 1.8% for the week. While Sable Island deliveries to Region 1 (NE) increased 21%, the TCPL major Region 1 (NE) deliveries via Niagra, Iroquois and Chippawa were down 800 MMCF from last week.  Since Sable Island was up 600 MMCF over last week, it suggests some displacement of Western Canadian gas by Atlantic provinces production.

Once again Peoples Gas Light and Nicor receipts from Alliance PL were over 33%  (1.8 BCFW) of dry gas deliveries net of Vector PL reshipments and Aux Sable shrinkage due to liquids removal.  There is every reason to believe this is ending up in Chicago storage, which goes a long way towrd explaining AGA weekly changes.

### Table F

###### Comparison of Current and Prior Year Weekly Changes

| Comparison of Current and Prior Year Weekly Changes |  |  |  |
| --- | --- | --- | --- |
| U.S. Change | U.S. Change |  | Week Starting Gross Gas Inventory |
|  | BCFW | BCFD | BCF |
| Current Week 2001 | 84.1 | 12.0 | 6,072 |
| Year Ago 2000 | 67.0 | 9.6 | 6,404 |

This week’s fill rate was 25% higher than last year’s at the same time, but inventories this year are 5.2% lower than last year, same time.

Table G

Weekly Change Factor as Per cent of Week's Starting Inventory

| Weekly Regional Change Factor as Per Cent of Starting Inventory |  |  |
| --- | --- | --- |
| Region | % Change This Week | % Change Last Week |
| U.S. | 1.4 | 1.4 |
| Region 1 NE | 2.4 | 3.0 |
| Region 2 ENC | 1.2 | 1.5 |
| Region 3 WNC | 1.0 | 0.7 |
| Region 4 SE | 1.2 | 0.8 |
| Region 5 SW | 1.4 | 1.0 |
| Region 6 MTN | 0.8 | 0.5 |
| Region 7 PAC | 1.1 | 0.7 |

Table H

Imputed Industrial Natural Gas Demand

| Direct Gas Use in Selected Product Manufacturing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Industry | Gas Application | NASIC CODE | Units | Reported Production | Imputed Gas Use BCFM |
| Petroleum Refining | Process Heat | 4227 |  |  |  |
| Ethylene | Furnace Heat |  |  |  |  |
| Ammonia/Methanol | Feedstock |  |  |  |  |
| Chlor-Alkali | Electrochem/Evap | 4226 |  |  |  |
| Pulp and Paper | Process Heat | 322 |  |  |  |
| Automotive | Metals | 4211 |  |  |  |
| Glass | Flat, Container |  |  |  |  |
| Table Total | NA | NA | NA | NA |  |

We are creeping up on this table – many traditional data sources are no more, so we have to develop alternates to ensure consistent data.

U.S. Ammonia production is off due to plant closures where gas costs made manufacture

uneconomic where the ammonia is not a precursor (raw material) for a downstream plant, such as urea. Whether shut down facilities will come back up is a question, because the plants are hard to mothball and most are fairly old. Imported ammonia is inexpensive and in ample supply, so there is little reason to struggle with old, inefficient plants if it can be imported for less.

Methanol is pretty much a dead duck, except for captive, downstream plants that are kept open partly for competitive supply reasons. Methanex (Canadian)  plants have beeen mothballed or closed because of unfavorable production costs, so it is a North American phenomenon, not just U.S..

Metals, especially Aluminum, is beginning to come back in Texas, so gas use can as well. Micro steel mills are significant power as well as users, so often have cogen facilities on site. Their economics are primarily impacted by imported steel prices and alleged dumping.

Glass containers are having a tough time because of aluminum cans and plastic (PETN) bottles, but flat glass output is tied to home construction and automobiles.

Pulp and paper is cyclical and tends to track economic activity; gas use is primarily in pulp production steps which do not have any alternative energy source.

The dominant inorganic chemical(s) that are major gas consumers are caustic soda and chlorine, which are produced as electro chemical equivalents in the same equipment. The gas is used to generate power and steam; the power dissociates brine, the steam is used to concentrate the caustic and in other on site units.

Petroleum refining gas uses are for process heat and as feedstock to hydrogen units.  Petrochemical gas needs are for process heat and feedstocks. While some alternates to gas can be used, they are usually neither competitive nor long term solutions.

Automotive manufacturing gas demands are for metal treating, space heating of factories and drying. We just used gross demand data to ratio gas use to production rates. It’s not pretty, but it seems to work.

Graphics Discussion

2/04/200

We added a special chart to show you what EIA said about average daily June injections since 1990 – the reason for this is to compare AGA average weekly numbers with another public source. As can be seen, since 1997, EIA reported average daily June injections have tailed off dramatically.  This suggests that storage is being operated quite differently than traditional fill/pull cycles used by LDC’s.

AGAWG inventories are now exactly on those of GasFax®; AGA weekly injections seem to be paralleling those of GasFax®, except at 15 BCFD rather than 12 BCFD.

GasFax® GG inventories are still about 150 BCF lower than the three year average in spite of the heroic injection efforts suggested by AGA. The EIA STIFs model is now right on GasFax® 01, for whatever comfort that gives us. Both are below the EIA three year average.

GasFax®

This week’s topic is about LNG imports - how much impact do they  have on Apparent Demand calculations?  In a word, “some”. The basis for this conclusion is that “stranded gas” production from new oil and gas discoveries in Africa and Asia had no home unless converted to LNG or gas derivative, such as ammonia. The alternatives were to flare,  re-inject the associated (co-product) gas from crude production or not produce gas well gas at all. These stark realities put a new variable in LNG economics - the raw gas itself essentially had zero, or even negative, value without a market. “Net back” sales based upon production point value after conversion to LNG,  transportation, reception and distribution costs then became economically feasible. (The first U.S. LNG program assumed POS (Point of Shipment) fixed prices to which all subsequent costs were added and the resulting delivered costs blended into the FERC rate base.)  The difference showed the effect of changing to market driven economics from cost determined, regulatory rates.

The purpose of this week’s imports review is to define the relationships between North American pipeline gas supplies, LNG imports and apparent demand. The question was how much gas was exported vs total imports in all forms, including LNG.  Although exports were not trivial, Canadian gas re-imports from the U.S. were less as a percentage than anticipated. Mexican exports were essentially lost in the round off and had no real impact, even though they were twice as large in percentage as Canadian re-imports.

The table shows us three important facts:

- Pipeline gas imports will remain the dominant inland form

- LNG will  be the fastest growing source of gas to the U.S.

- LNG will serve coastal U.S. markets primarily, where demand growth is highest

LNG exports are exclusively from Alaska, and almost entirely to Japan. Until an Alaskan Pipeline is built to carry Alaskan gas to the lower 48 states, LNG will be the only way for gas to be shipped from Alaska. Obviously, LNG can be transported from Alaska to California, but the economics become far more complex and outside this discussion. (The significance is that Chevron has announced a possible California LNG terminal - it could be served by Indonesian LNG as well as Alaskan)

The question raised was whether LNG imports play a significant role in determining whether Contiguous U.S. Apparent Demand calculations were affected by LNG imports, and the answer is “yes”. LNG imports are growing fast, with the total right now past round off volumes and likely to expand significantly. Why now? Because natural gas has become a major world free market energy form and stranded gas has grown exponentially.

The only options were conversion to derivatives, whose outlets were limited; LNG, whose logistics were costly, flaring or re-injection. The only choice that promised an acceptable producer return was LNG, and it opened up world scale opportunities if the concept of zero alternate value was accepted by producer states.  Given the options, net back sales were logical.

Shipping economics also changed for the better, since ship construction costs have dropped significantly. (Fuel expenses are essentially zero because the ships use the LNG “boil off”  or vaporization in transit for engine fuel). Ditto debits for conversion of gas to LNG, about 20% by volume of plant feed. Thus, operational expenses are not factors, and only capital expenses and producer state taxes or charges impact LNG supplier economics. Obviously unit capital charges are affected by economies of scale, so the more LNG that is produced and shipped from a given LNG plant, the lower fixed costs are as a proportion of all expenses.

The fact that LNG is a coastal energy form, by definition, means that receiving terminals can be located wherever permits can be gained for them. U.S. demographics show that the highest population concentrations and growth are in coastal regions, especially in so called sun belt states. People equal gas and energy demands, so LNG suddenly was re-discovered when net back economics became acceptable to producer states.

Returning to LNG as a factor in Contiguous U.S. gas import balances,  Canadian gas pipeline shipments accounted for 94% of all U.S. gas imports in 2000, while LNG amounted to 6%. Total U.S. gas exports were 4.5% of total imports (not in table), disregarding Alaskan LNG. Natural gas exports to Mexico were 64% of the total, with Canadian re-imports 36% in 2000. The size of exports to Mexico was not expected, and the growth rate of 79% since 1995 an even bigger surprise.

The bottom line? LNG will be added to Contiguous U.S. natural gas supplies in calculating apparent demand and historical data will have the adjustment made to ensure consistency.

| Comparison of U.S. Annual Natural Gas Imports and Exports by Type and Selected Sources |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCFY |  |  |  |  |  |  |  |  |  |
|  |  | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | % Change 1995-2000 | % of Total, 2000 |
| Total | All Imports | 2841.0 | 2937.4 | 2994.2 | 3152.1 | 3536.4 | 3806.8 | 34.0 | 100.0 |
| Gross Pipeline Imports | Canada, Mexico | 2823.1 | 2897.1 | 2916.4 | 3066.6 | 3373.0 | 3575.0 | 26.6 | 93.9 |
| Pipeline Exports | Canada | 27.5 | 51.9 | 56.4 | 39.9 | 38.5 | 62.8 | 128.4 | 36.4 |
| Pipeline Exports | Mexico | 61.3 | 33.8 | 38.4 | 53.1 | 61 | 109.7 | 79.0 | 63.6 |
| LNG Imports | All | 17.9 | 40.3 | 77.8 | 85.5 | 163.4 | 231.8 | 1193.7 | 6.1 |
| LNG Exports | All | 88.8 | 85.7 | 94.8 | 93.0 | 99.5 | 172.5 | 94.3 | 100.0 |
| Net Imports | Contig U.S. | 2752.2 | 2851.7 | 2899.4 | 3059.1 | 3436.9 | 3634.3 | 32.0 | 95.5 |