Thought Provoking Stuff

Black tank:

- Add a gallon of water before use (avoid the dreaded “poop pyramid”)

- Use deodorizer

- Empty black tank before grey tank

- Use a clean out wand during/after dumping

Grey tank:

- Older Casitas have grey tank exit plumbing running slightly “up-hill”

- Stack leveling pads (cut in half) under curb side trailer wheel when visiting the dump site

Water heater:

- Always (ALWAYS) insure that the water heater is full before activating (prevent electric heating element burn-out)

- Electric:

  - The primary electric switch is outside on the lower left side of the water heater behind the (gas) venturi tube
The secondary switch (if installed via a modification) is inside the travel trailer
Both switches must be “on” to run the water heater on electric

- Propane:

  - Use when electric (shore power) is not available

  - Interior propane side light illuminates only when water heater initiates ignition cycle

- Two thermostats: one for electric, another for propane

Fridge:

- Three way: 120v ac, propane or 12v dc

- Works flawlessly on 120v ac and propane operation

- Conventional wisdom(?):

  - Absorption fridge (ammonia based) requires(?) that the trailer be level (front to back and side to side)

  - 12v operation seems to be anemic (and might increase demand on battery or tow vehicle alternator)

- Door shelves are not designed to hold heavy items while driving ; especially canned soda and beer

Electric:

- 120v ac:

  - 30 amp cord with connector

  - circuit breakers

  - air conditioner

  - interior outlets and exterior outlet

- 12v dc:

  - automobile type fuses

  - supplied by battery which is maintained by the converter (shore power or generator) or solar or tow vehicle

| - lights - fridge air circulation - fridge temperature control - fridge 12v operation - fridge propane ignition - break-away switch (for electric trailer brakes) | - furnace thermostat - furnace propane ignition - furnace fly switch - furnace blower - solar controller ## - solar bluetooth module ## - USB Outlets ## | - stove vent - bath vent - water pump - water heater electric thermostat - water heater propane thermostat - water heater propane ignition - propane detector ## |
| --- | --- | --- |

- Battery Isolation:  ## use battery switch or anderson disconnect to isolate the battery when not camping or driving

- Converter

  - converts 120v ac to 12+v dc

  - insure that converter controls provide for wet-cell/AGM/Lithium charging

  - if interior lights do not work when connected to shore power (or generator) and the battery is disconnected and the tow vehicle is disconnected and solar is disconnected, then the converter is “dead”

- Inverter (optional equipment)

  - converts 12v dc to 120v ac

  - Requirements calculator:

    - calculate amperage for each appliance using formula: I = P ÷ E or conversely P = I x E
where I = amps and P is expressed in watts and E = 120 (volts)

    - sum up the amps from each appliance to figure out which appliances can be run together or separately

- Solar (optional equipment):

  - Used when “dry camping” or “boondocking”

  - Primary use is to “top off” the battery as it becomes discharged

- Generator [or Inverter Generator] (optional equipment):

  - Used when “dry camping” or “boondocking”

  - Requirements calculator (same as inverter above)
Nominal max required for a Casita is 3,600 watts (30 amps)

Winterizing:

- antifreeze (pink stuff) method vs. blow-out method

  - antifreeze method requires un-winterizing process

  - blow-out method requires use of blow-out plug and air compressor (or tire inflator)

- both methods require use of the “bypass valve”

- blow out method:  Casita – How to Winterize your Travel Trailer by RV Adventures

Battery:

- wet cell vs. AGM vs Lithium

- CCA does not pertain to a travel trailer

- Deep Cycle (reserve capacity in amp hours) is the significant measure

- Disconnect switch or Anderson disconnect is needed when storing trailer for long periods to isolate parasitic loads (propane detector, solar controller, blue-tooth devices, etc.) [see Battery-Isolation above]

- Lithium considerations:

  - cell balancing feature

  - internal heater?

  - charging control (DC-DC converter) to protect tow vehicle alternator from burn-out

Surge Protector

- Always use when camping to protect trailer from surges and ground faults

Water:

- Always use a pressure reducer AND an inline water filter when connected to “City Water”

- Use a water tank filler device when filling the fresh water tank

LED lights:

- Best to use bulbs that are rated 9v – 15v (voltage levels from battery and converter)

Propane Usage:

- For dual tanks, always turn one tank off

- The center position of the propane valve connects both tanks together;
it does not auto-switch from one tank to the other when one tank empties

- Refill the “primary tank” when manually switching to the “secondary tank”

Stabilizers

- Two stabilizers are provided at the rear of the trailer just in front of the rear bumper

- This video describes the procedure to set and stow the stabilizers

Trailer hook-up process to tow vehicle:

- adjust trailer tongue height approximately ½" higher that tow vehicle hitch ball

- In lieu of having a spotter to assist, use a magnet trailer alignment kit (Harbor Freight 69778 or equivalent)

  - put one “marker” on the trailer tongue

  - put the other “marker” on top of the tow vehicle hitch ball

  - align the tow vehicle and back it toward the trailer until the trailer tongue knocks the marker” on the hitch ball askew

- After dropping and latching the tongue onto the hitch ball:

  - Using the tongue jack, raise the tongue until the tow vehicle also raises (confirms hookup)

  - If using a weight distribution hitch, with the tow vehicle raised, set and adjust weight distribution chains (or trunnion bars)

  - Using the tongue jack, lower the trailer tongue until the tongue jack is completely compressed and in a stored position

- Hook up safety chains by crossing them from the tongue to the tow vehicle hitch receiver

- Hook up break-away cable

- Attach sway bar

Window-frame Maintenance:

- Periodically insure that the weep holes on the window frames are not clogged by using three-four feet of
.095 weed-eater trimmer line to dislodge any dust/silt that might have accumulated inside the channels

Information:

- Casita Travel Trailer A-Z Owner’s Guide by Eileen Glick also on Amazon
I got the digital version from Eileen’s website and my license is for personal use on each/every device I own

- Don Dayton’s Casita Mods a great collection of “how to” modifications in the form of cookbook pictures

- FiberglassRV.com

- Escape Forum

- The Handy Camper

- Little House Customs

- Annual Rally Maps:  <2025> and <2024>

Black Tank Dumping:

Although I have never used the toilet in my Casita, here are some useful stories that I have chanced upon:

- Empty your waste tanks cleanly and easily

- Black water spill?

- Movie clip:  RV – The Poop Geyser Scene (Robin Williams)